EMBEDDED SYSTEMS INTERNSHIP  |  FIRMWARE DEVELOPMENT TASK

LED-CTRL

Interrupt-Driven LED Controller with UART Console, Temperature Alert and Watchdog

Firmware design, simulation, testing and documentation report

| Prepared by | Saurav Pardeshi |
| --- | --- |
| Programme | BTech, Electronics and Telecommunication Engineering |
| Task | Design and implement firmware on a simulated embedded system; test it and document the process |
| Deliverables | led_controller.c (single file, README embedded), this report, captured test logs |
| Language / tools | C99, gcc, AddressSanitizer, UndefinedBehaviorSanitizer |
| Version / date | 1.0 ¦ October 2026 |

All test results in this report were produced by running the submitted source file; the logs in the appendices are unedited program output.

# Contents

1  Introduction and objectives

2  Application and requirements

3  Firmware architecture

4  Implementation details

5  Simulation environment

6  How to compile and run

7  Test strategy and test plan

8  Test results

9  Debugging log and lessons learned

10  Quality assurance

11  Limitations and future work

12  Development process

13  Conclusion

Appendix A  Complete self-test output

Appendix B  Demo and interactive session logs

Appendix C  Complete source code

# 1  Introduction and objectives

This report documents a firmware project built for the embedded-systems internship task code development and hands-on implementation. The task asks for firmware running on a simulated embedded system, for example an LED controller using timer interrupts or UART communication, together with inline documentation, a README, a test record and evidence of a methodical development process.

Rather than choosing only one of the suggested examples, the project combines three of them in one small but realistic product, LED-CTRL: a timer-interrupt-driven LED controller with a UART command console and an analog temperature sensor with an over-temperature alert, protected by a watchdog. This exercises the main building blocks of real firmware: interrupts, a time base, a peripheral driver layer, data buffering, a text protocol, sensor signal processing, fault handling and recovery.

## 1.1  Objectives

- Implement interrupt-driven firmware in C with a clear separation between ISRs, main loop and hardware access.

- Provide a software model of the hardware so that the firmware can be compiled, run and tested without a board.

- Verify correctness with an automated test suite that measures timing, accuracy, robustness and fault recovery.

- Document the code (inline comments and README), the architecture, the testing and the debugging process.

## 1.2  Deliverables

| Deliverable | File | Purpose |
| --- | --- | --- |
| Source code | led_controller.c | Single C file: firmware, hardware simulator and test suite; README and inline comments included at the top and throughout |
| Report | LED-CTRL_Firmware_Report.docx | This document: design, implementation, testing, debugging log, appendices with full output and source |
| Test log | test_results.txt | Unedited output of the build, self-test, sanitizer, demo and mutation-test runs |

# 2  Application and requirements

## 2.1  Why this application

The three example applications in the brief (LED with timer interrupts, UART communication, sensor reading) share the same architecture pattern: an interrupt produces an event, a main loop consumes it, and a driver layer touches the hardware. Combining them shows that the architecture scales beyond a toy blinker. A temperature alert gives the LED a real purpose (it is the alarm indicator), and the UART console makes the system observable and testable from outside, which is how embedded systems are commonly debugged in the field.

## 2.2  System description

The simulated board has one PWM-capable LED, a UART at 115200 baud 8N1, an LM35-type analog temperature sensor (10 mV per degree Celsius) connected to a 12-bit ADC with a 3.3 V reference, an independent hardware watchdog with a 2 s timeout, and a small backup register that survives resets. The user controls the system with text commands such as LED BLINK 250 or THRESH 30.

## 2.3  Requirements

Table 1 – Functional (F) and non-functional (N) requirements

| ID | Requirement |
| --- | --- |
| F1 | The LED shall support four modes selectable over UART: OFF, ON, BLINK with a given period, and BREATHE (smooth fade). |
| F2 | LED brightness shall be adjustable from 0 to 100 % and shall scale all modes. |
| F3 | BLINK period shall be accepted from 50 to 5000 ms (default 500 ms) with 50 % duty and no drift, driven by the timer interrupt. |
| F4 | The firmware shall sample the temperature every 100 ms, filter it with an 8-sample moving average and report it with 0.1 °C resolution. |
| F5 | An over-temperature alert (default 35 °C, configurable 10 to 100 °C) shall override the LED with a fast 100 ms blink and clear only after the temperature falls 2 °C below the threshold (hysteresis). |
| F6 | The UART command parser shall be case-insensitive, validate every argument and answer with OK ... or a specific ERR ... message. |
| F7 | A STATUS command shall report mode, brightness, temperature, threshold, alert state, uptime and health counters. |
| F8 | A watchdog shall reset the system if the main loop stops running, and the firmware shall recover to a known state. |
| F9 | At every boot the firmware shall print its version, the reset cause and a count of watchdog resets that survives the reset. |
| N1 | Interrupt handlers shall be minimal and non-blocking; all processing shall be deferred to the main loop. |
| N2 | No dynamic memory allocation and no floating-point arithmetic in the firmware part. |
| N3 | The firmware shall never crash or lock up on malformed input, over-long lines or receive-buffer overflow. |
| N4 | All hardware access shall go through a small HAL so that the firmware can be ported to a real MCU. |
| N5 | The code shall be thoroughly commented and shall compile warning-free with strict compiler flags. |
| N6 | The system shall be testable without hardware using an automated suite with a pass/fail exit code. |

# 3  Firmware architecture

## 3.1  Overview

The firmware follows the classic interrupt plus superloop pattern. Two interrupt service routines do the absolute minimum and hand work to the main loop through volatile variables: SysTick increments the millisecond counter and raises a tick flag, and the UART receive interrupt stores the incoming byte in a ring buffer. The main loop function app_poll() then performs all the real work in a fixed order: process pending ticks, process received characters, and kick the watchdog.

Figure 1 – Firmware architecture: interrupts, cooperative main loop, hardware abstraction layer and simulated hardware.

## 3.2  Modules and responsibilities

Table 2 – Key functions (all in led_controller.c)

| Function | Type | Responsibility |
| --- | --- | --- |
| SysTick_Handler() | ISR, 1 kHz | Increments the millisecond counter and the pending-tick counter. |
| USART2_IRQHandler() | ISR | Reads the received byte and pushes it into a 64-byte ring buffer; counts bytes lost on overflow. |
| app_poll() | Main loop step | Services every pending tick, drains the UART buffer, kicks the watchdog only while alive. |
| on_tick_1ms() | 1 ms task | Runs the LED engine every ms and the sensor sampling every 100 ms. |
| led_engine_step() | 1 ms task | Computes the LED level for OFF / ON / BLINK / BREATHE and writes the HAL only on change. |
| sensor_sample() | 100 ms task | ADC read, conversion to 0.1 °C, 8-sample moving average, alert with hysteresis. |
| uart_rx_service() | Main loop | Assembles lines: handles CR/LF, backspace, TAB, over-long lines. |
| process_line() | Main loop | Upper-cases, tokenises, validates and executes commands; replies OK or ERR. |
| fw_boot() | Startup | Initialises all state and prints the boot banner with reset cause and counter. |
| hal_*() | HAL | Ten small functions hiding LED, UART, ADC, watchdog, backup register and interrupt control. |

## 3.3  Concurrency model

There are only two contexts: interrupt and main loop. Shared data is deliberately minimal and each variable has exactly one writer. The ring-buffer head index is written only by the receive ISR and the tail index only by the main loop, so a single-producer, single-consumer buffer needs no lock. The pending-tick counter is touched by both contexts, so the main loop reads and decrements it inside a short critical section (CRIT_ENTER() / CRIT_EXIT(), which map to __disable_irq() and __enable_irq() on a Cortex-M). All shared variables are declared volatile.

## 3.4  Timing and resource analysis

The following figures come from calculation and from the measurements in Section 8; instruction-count figures are estimates for a typical 16 MHz Cortex-M core and would be confirmed on real hardware.

| Item | Value | Comment |
| --- | --- | --- |
| UART byte time at 115200 baud, 8N1 | 86.8 µs | 10 bits per byte |
| Time to fill the 63 usable ring-buffer slots | 5.47 ms | Main loop runs about every 1 ms, so there is roughly 5x margin |
| Longest accepted command line (40 chars) | 3.47 ms | Fits easily between two main-loop passes |
| SysTick ISR cost (estimate) | < 30 cycles | About 0.2 % of the 16 000 cycles per millisecond at 16 MHz |
| Watchdog timeout vs loop period | 2000 ms vs 1 ms | A hang is detected within 2 s; false resets are practically impossible |
| Static RAM of firmware data (estimate) | about 0.2 KB | 64 B ring + 41 B line buffer + 16 B filter window + scalars; no heap |

## 3.5  Design decisions

- Integers only. Temperatures are kept in tenths of a degree. For an LM35 at 10 mV/°C one millivolt equals 0.1 °C, so the converted value in millivolts is directly the temperature in tenths, avoiding any floating point.

- Tick counting instead of work in the ISR. The ISR only counts; if the main loop is ever late, the tick counter accumulates and the loop catches up while recording an overrun counter (visible in STATUS).

- Write the LED only when the level changes. This keeps the peripheral traffic low and makes edge timing measurable in the simulator.

- Strict command validation. Numbers are parsed with strtol and checked for trailing characters and range; the reply distinguishes bad argument from out of range.

- Hysteresis and averaging. An 8-sample average rejects single-sample glitches and a 2 °C hysteresis prevents the alert from toggling when the temperature hovers around the threshold.

- Watchdog kicked from one place only. The kick sits at the end of the main-loop pass, so it happens only if ticks and UART were both serviced.

# 4  Implementation details

## 4.1  Time base and interrupt handlers

The two ISRs are intentionally tiny. They use the real CMSIS handler names, so on an STM32 they can stay unchanged.

void SysTick_Handler(void)

{

g_ms++;

g_ticks_pending++;

}

void USART2_IRQHandler(void)

{

uint8_t b = hal_uart_read_rx();

uint8_t next = (uint8_t)((rx_head + 1u) & (UART_RX_BUF_SIZE - 1u));

if (next == rx_tail) {          /* buffer full: drop byte, count the loss */

rx_overflow++;

return;

}

rx_buf[rx_head] = b;

rx_head = next;

}

## 4.2  Receive path and line assembly

Characters flow ISR → ring buffer → uart_rx_service() → line buffer → process_line(). The ring size is a power of two so that wrapping is a bit-mask rather than a division. One slot is kept empty to distinguish full from empty, so 63 bytes are usable. The line assembler accepts printable ASCII and TAB, handles backspace (0x08 and 0x7F), ignores other control characters and treats CR, LF or CRLF as the end of a line. A line longer than 40 characters is discarded until the end-of-line and answered with ERR LINE TOO LONG, so a flood of garbage cannot corrupt memory or block the console.

## 4.3  Command grammar

Table 3 – UART command set

| Command | Arguments | Reply (success) | Typical errors |
| --- | --- | --- | --- |
| HELP | - | COMMANDS: ... | - |
| VER | - | OK VER 1.0.0 | - |
| LED ON / LED OFF | - | OK LED ON / OFF | ERR BAD ARG, ERR MISSING ARG |
| LED BLINK [ms] | 50 to 5000, default 500 | OK LED BLINK 200 | ERR RANGE 50-5000 |
| LED BREATHE | - | OK LED BREATHE | - |
| BRIGHT n | 0 to 100 | OK BRIGHT 40 | ERR RANGE 0-100, ERR BAD ARG, ERR MISSING ARG |
| TEMP | - | OK TEMP 25.0C | ERR NO DATA (before first sample) |
| THRESH n | 10 to 100 (°C) | OK THRESH 28 | ERR RANGE 10-100, ERR MISSING ARG |
| STATUS | - | STATUS mode=... temp=... alert=... | - |
| HANG | - | OK HANG ... | Debug command that stops the main loop to demonstrate the watchdog |
| (anything else) | - | - | ERR UNKNOWN CMD, ERR TOO MANY ARGS, ERR LINE TOO LONG |

## 4.4  LED engine

led_engine_step() runs once per millisecond and computes the output level from a phase counter phase_ms that is restarted whenever the mode changes. With period the full cycle in milliseconds and B the brightness:

- ON: level = B.   OFF: level = 0.

- BLINK: level = B while phase_ms < period/2, otherwise 0; phase_ms wraps at period. The rising edges are therefore exactly period ms apart.

- BREATHE: a triangle tri = phase for phase < 1000 and 2000 − phase otherwise (range 0 to 1000); level = tri × B / 1000, giving a 2 s up-and-down fade whose peak equals the brightness.

- Alert override: while the alert is active the engine ignores the user mode and uses BLINK with a 100 ms period.

## 4.5  Temperature pipeline and alert logic

Every 100 ms the 12-bit code is converted with a rounded integer formula, mV = (raw × 3300 + 2047) / 4095, which equals tenths of a degree for the LM35. The value is stored in an 8-element window and the average is recomputed. The alert is judged only once the window is full, so start-up transients cannot cause a false alarm.

if (t_cnt == AVG_WINDOW) {      /* judge only once the window is full */

int32_t hi = thresh_c * 10;

if (!alert_active && t_avg_c10 >= hi) {

alert_set(true);

} else if (alert_active && t_avg_c10 <= (hi - HYST_C10)) {

alert_set(false);

}

}

## 4.6  Watchdog and recovery

The independent watchdog has a 2 s timeout. app_poll() kicks it at the very end of each pass, so it is refreshed only if the loop is running. The HANG command sets a flag that makes app_poll() return immediately, which emulates a stuck main loop while the interrupts continue to fire. After 2 s the simulated watchdog resets the system; the reset cause and a counter stored in the backup register are reported at the next boot, and the firmware returns to its default state and accepts commands again.

## 4.7  Error-handling policy

Table 4 – Faults and responses

| Fault | Detection | Response |
| --- | --- | --- |
| Unknown or malformed command | Parser | Specific ERR reply; state unchanged |
| Argument out of range or not a number | strtol + range check | ERR RANGE / ERR BAD ARG; previous value kept |
| Line longer than 40 characters | Line assembler | Discard to end of line, ERR LINE TOO LONG |
| UART receive-buffer overflow | ISR finds buffer full | Drop byte, increment rxovf counter shown in STATUS |
| Main loop late by more than one tick | Pending-tick counter > 1 | Catch up; increment tickovr counter |
| Single bad ADC sample | Moving average | Impact divided by 8; no false alert |
| Main loop hang | Hardware watchdog | Reset after 2 s, reset cause and counter reported |

# 5  Simulation environment

Section E of led_controller.c is a software model of the board. It owns the "hardware" state (LED duty, UART data register, ADC input, watchdog counter, backup register) and advances the whole system in exact one-millisecond steps. In each step it advances time, checks the watchdog, calls SysTick_Handler() as the timer hardware would, and then gives the main loop a chance to run with app_poll(). Bytes sent to the simulated UART trigger USART2_IRQHandler() one byte at a time, spaced by 87 µs of simulated time to mimic 115200 baud.

Table 5 – What the simulator can inject

| Hook | Purpose |
| --- | --- |
| Real temperature (sim.temp_c10) | Drives the LM35/ADC model; ramps, steps and constant values |
| One-shot ADC glitch (sim.spike_raw) | Tests the moving-average filter |
| ±1 LSB deterministic ADC noise | Realistic quantisation noise; reproducible because the generator is seeded |
| UART burst (sim_uart_burst) | Delivers bytes faster than the main loop can drain them to force overflow |
| HANG command | Stops the main loop so that the watchdog has to recover the system |
| LED edge and level recorder | Timestamps every LED edge so that blink periods and duty can be measured exactly |
| CSV trace (--csv) | Records LED level, temperature and alert state every millisecond for plotting |

## 5.1  Fidelity and limits

The simulation is cycle-coarse: it models behaviour at millisecond granularity and the ISRs run between main-loop passes rather than pre-empting it in the middle of a statement. It therefore verifies logic, timing relationships and fault handling, but it cannot reveal true pre-emption races, flash wait states or analog effects. The firmware is written as if interrupts could occur at any time (volatile variables, single-writer indices, short critical sections), which is what a real port requires.

## 5.2  Porting to a real STM32

| HAL function | Real implementation on STM32 |
| --- | --- |
| hal_led_set_level() | __HAL_TIM_SET_COMPARE(&htim3, TIM_CHANNEL_1, ...) for a PWM duty cycle |
| hal_uart_putc() | HAL_UART_Transmit() or a write to USART2->TDR |
| hal_uart_read_rx() | Read USART2->RDR inside the RXNE interrupt |
| hal_adc_read() | HAL_ADC_GetValue() after a conversion |
| hal_wdg_kick() | HAL_IWDG_Refresh() |
| hal_reset_cause() | Check the IWDGRSTF flag in RCC->CSR |
| hal_persist_get/set() | RTC backup register |
| hal_irq_disable/enable() | __disable_irq() / __enable_irq() |

The same hal layer could also be targeted at an instruction-set simulator or board simulator such as Wokwi or Renode as a next step; only these ten functions would change.

# 6  How to compile and run

## 6.1  Requirements

Any C99 compiler (gcc or clang) on Linux, macOS, or Windows with MinGW or WSL. No external libraries and no hardware are needed.

## 6.2  Build

gcc -std=c99 -Wall -Wextra -Wpedantic -Werror -O2 led_controller.c -o ledctl

# optional: sanitizer build used during development

gcc -std=c99 -Wall -Wextra -Wpedantic -g -O1 -fsanitize=address,undefined \

led_controller.c -o ledctl_san

## 6.3  Run

| Command | What it does |
| --- | --- |
| ./ledctl --selftest | Runs the 16 test cases (97 checks); exit code 0 means all pass |
| ./ledctl --selftest --verbose | Same, and also prints the UART traffic with timestamps |
| ./ledctl --demo | Scripted scenario: all LED modes, errors, temperature ramp with alert, watchdog reset |
| ./ledctl --demo --csv out.csv | Demo plus a 1 ms CSV trace (t_ms, led_pct, temp_true_c10, temp_avg_c10, alert) |
| ./ledctl --interactive | Type UART commands yourself; directives: @wait <ms>, @temp <degC>, @quit |

## 6.4  Example interactive session

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

Interactive mode. Type UART commands. Directives: @wait <ms>, @temp <degC>, @quit

[      0 ms] RX> LED BLINK 250

[      1 ms] TX> OK LED BLINK 250

[   1053 ms] SIM> temperature set to 41.0 degC

[   1500 ms] TX> ALERT ON temp=35.0C thresh=35C

[   2553 ms] RX> STATUS

[   2553 ms] TX> STATUS mode=BLINK period=250 bright=100 led=0 temp=41.0C thresh=35C alert=1 up=2553ms rxovf=0 tickovr=0 wdg_resets=0

[   2606 ms] RX> BRIGHT 300

[   2606 ms] TX> ERR RANGE 0-100

# 7  Test strategy and test plan

Testing is built into the same source file (--selftest) so that anyone can reproduce it with one command. The suite combines several test types, each aimed at a different class of defect:

- Unit tests of pure functions and the ring buffer (TC-15, TC-16).

- Functional tests of every command and mode (TC-01, 03, 04, 06, 12, 13).

- Timing tests that measure LED edge timestamps to the millisecond (TC-02, TC-05).

- Accuracy tests of the ADC-to-temperature chain at five temperatures (TC-09).

- Robustness and fault-injection tests: bad input, overflow, glitches, hang (TC-07, 08, 10, 14).

- Boundary tests: exact threshold and hysteresis edges, minimum and maximum arguments (TC-04, 05, 11).

Table 6 – Test plan and traceability

| ID | Test case | Type | Covers | Checks | Result |
| --- | --- | --- | --- | --- | --- |
| TC-01 | Boot banner, initial state | Functional | F9 | 5 | PASS |
| TC-02 | SysTick time base accuracy and tick servicing | Timing | N1 | 3 | PASS |
| TC-03 | Basic UART commands, case-insensitivity | Functional | F1, F6 | 4 | PASS |
| TC-04 | BRIGHT command and argument validation | Functional / validation | F2, F6 | 8 | PASS |
| TC-05 | Blink timing (timer interrupt accuracy) | Timing | F1, F3 | 8 | PASS |
| TC-06 | Breathe (triangle PWM fade) | Functional | F1, F2 | 6 | PASS |
| TC-07 | Input robustness: unknown, blank, tabs, backspace, too many args | Robustness | F6, N3 | 9 | PASS |
| TC-08 | UART overflow and over-long line handling | Robustness / fault injection | N3 | 5 | PASS |
| TC-09 | Temperature measurement accuracy (ADC + filter) | Accuracy | F4 | 5 | PASS |
| TC-10 | Moving average suppresses a single-sample glitch | Fault injection | F4, F5 | 2 | PASS |
| TC-11 | Over-temperature alert, LED override and hysteresis | Functional / boundary | F5 | 11 | PASS |
| TC-12 | THRESH command | Functional / validation | F5, F6 | 7 | PASS |
| TC-13 | STATUS report and TEMP before first sample | Functional | F7 | 5 | PASS |
| TC-14 | Watchdog recovers a hung main loop | Fault injection / recovery | F8, F9 | 9 | PASS |
| TC-15 | Ring buffer unit test (wrap-around, full, empty) | Unit | N3 | 4 | PASS |
| TC-16 | ADC conversion unit test | Unit | F4 | 6 | PASS |
|  | Total |  |  | 97 | ALL PASS |

# 8  Test results

## 8.1  Summary

The final run of ./ledctl --selftest executed 16 test cases with 97 checks: 97 passed, 0 failed. The complete unedited output is in Appendix A. The same suite and the demo also run without any report under AddressSanitizer and UndefinedBehaviorSanitizer (Section 10).

## 8.2  Measured behaviour

Table 7 – Measurements taken from the simulation

| Quantity | Requirement / expectation | Measured |
| --- | --- | --- |
| SysTick time base (10 000 interrupts) | 10 000 ms | 10 000 ms exactly, 0 tick overruns |
| Blink period, command LED BLINK 200 | 200 ms, 50 % duty | Every rising-edge interval exactly 200 ms; on-time exactly 100 ms |
| Blink period, minimum LED BLINK 50 | 50 ms | Every interval exactly 50 ms |
| BREATHE peak position and level | Peak at 1.0 s of a 2 s cycle, equal to brightness | Peak 100 % at 997 ms after the first sample window (within the ±10 ms test tolerance); 50 % with BRIGHT 50 |
| Temperature accuracy at 0, 25.0, 37.5, 50.0, 99.0 °C | within ±0.2 °C | Error +0.1 °C at four points and 0.0 °C at one (ADC noise ±1 LSB included) |
| Glitch rejection (one sample reads 60 °C) | No false alert | Filtered value peaks at about 29.4 °C, no alert |
| Alert latency, 25 → 40 °C step | Within 1 s | 600 ms until ALERT ON |
| Alert clear latency, 40 → 32 °C step | Within 1.5 s | 700 ms until ALERT OFF |
| Alert LED pattern | 100 ms blink overriding user mode | Every interval exactly 100 ms |
| Hysteresis | On at ≥ 35.0 °C, off at ≤ 33.0 °C | Exactly 35.0 °C raises, 34.9 does not; 33.1 stays on, 33.0 clears |
| Receive overflow (100 bytes while loop busy) | No crash; losses counted | 37 bytes dropped and counted, line rejected, console usable afterwards |
| Watchdog recovery after HANG | Reset at about 2000 ms | Reset 1998 ms after the hang; banner shows reset=WATCHDOG resets=1; commands work again |

## 8.3  LED behaviour

Figure 2 was plotted from the 1 ms CSV trace of the demo run. The top panel shows the 400 ms blink (50 % duty), the middle panel the 2 s triangular fade, and the bottom panel the forced 100 ms alert blink at the user brightness of 60 %.

Figure 2 – LED output versus simulated time for BLINK, BREATHE and the alert override (from demo_trace.csv).

## 8.4  Temperature, filter and alert

Figure 3 shows the real temperature ramping from 25 to 40 °C and back to 28 °C, the filtered value used by the firmware, and the alert state. The filter lags the real value by about 0.4 s, the alert starts when the filtered value reaches 35.0 °C and clears only after it falls to 33.0 °C, demonstrating the 2 °C hysteresis.

Figure 3 – Real temperature, filtered value, alert thresholds and alert state during the demo.

## 8.5  UART session from the demo

An excerpt of the time-stamped UART log of the demo scenario (full log in Appendix B). Deliberate errors (BRIGHT 200, FLY) show the validation, and the last lines show the watchdog recovery.

$ ./ledctl --demo --csv demo_trace.csv

--- DEMO: scripted scenario on the simulated board ---

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[    500 ms] RX> HELP

[    500 ms] TX> COMMANDS: HELP VER LED ON|OFF|BLINK [ms]|BREATHE BRIGHT <0-100> TEMP THRESH <10-100> STATUS HANG

[    502 ms] RX> LED BLINK 400

[    503 ms] TX> OK LED BLINK 400

[   2505 ms] RX> LED BREATHE

[   2506 ms] TX> OK LED BREATHE

[   5508 ms] RX> BRIGHT 60

[   5509 ms] TX> OK BRIGHT 60

[   6511 ms] RX> LED ON

[   6512 ms] TX> OK LED ON

[   7014 ms] RX> LED OFF

[   7015 ms] TX> OK LED OFF

[   7517 ms] RX> TEMP

[   7517 ms] TX> OK TEMP 25.0C

[   7519 ms] RX> LED BLINK 1000

[   7521 ms] TX> OK LED BLINK 1000

[   7523 ms] RX> BRIGHT 200

[   7524 ms] TX> ERR RANGE 0-100

[   7526 ms] RX> FLY

[   7526 ms] TX> ERR UNKNOWN CMD

[   7528 ms] RX> STATUS

[   7529 ms] TX> STATUS mode=BLINK period=1000 bright=60 led=60 temp=25.0C thresh=35C alert=0 up=7529ms rxovf=0 tickovr=0 wdg_resets=0

--- temperature rises 25 -> 40 degC (threshold 35) ---

[  11800 ms] TX> ALERT ON temp=35.0C thresh=35C

[  14531 ms] RX> STATUS

[  14532 ms] TX> STATUS mode=BLINK period=1000 bright=60 led=60 temp=40.0C thresh=35C alert=1 up=14532ms rxovf=0 tickovr=0 wdg_resets=0

--- temperature falls 40 -> 28 degC ---

[  17700 ms] TX> ALERT OFF temp=32.8C thresh=35C

--- watchdog demonstration ---

[  20336 ms] RX> HANG

[  20337 ms] TX> OK HANG main loop stopped, watchdog will reset in 2000 ms

[  22337 ms] TX> BOOT LED-CTRL v1.0.0 reset=WATCHDOG resets=1

--- end of demo (simulated time 22942 ms) ---

# 9  Debugging log and lessons learned

The test suite was developed together with the firmware and was run repeatedly. The record below lists what actually went wrong and how each issue was found and fixed. It shows both a real firmware defect and defects in the tests themselves.

Table 8 – Test runs during development

| Run | Change before the run | Checks passed / failed |
| --- | --- | --- |
| 1 | First complete version of firmware and test suite | 91 / 2 |
| 2 | Fixed firmware defect D1 and test defect D2 | 93 / 0 |
| 3 | Added exact-boundary tests after mutation testing exposed gap D3 | 97 / 0 |

Table 9 – Defects found

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| D1 (firmware bug) | Test LED<TAB>OFF answered ERR UNKNOWN CMD | The line assembler accepted only printable characters 0x20 to 0x7E and silently discarded TAB, turning LED<TAB>OFF into LEDOFF, although the tokenizer was written to accept tabs | TAB (0x09) is now accepted into the line buffer; test TC-07 keeps it from returning |
| D2 (test bug) | Blink duty check failed: fall time minus rise time was not 100 ms | A debug print showed rise 202 fall 102: the edge recorder was cleared after the LED had already turned on, so the first recorded event was a falling edge and rise/fall pairs were shifted | Edge recording now starts before the command is sent |
| D3 (test gap) | Mutation M11 (>= changed to > in the threshold comparison) was not detected | The suite tested 34.0 °C and 40 °C but never exactly 35.0 °C | Added boundary tests with a noise-free ADC: 34.9 no alert, 35.0 alert, 33.1 stays, 33.0 clears |
| D4 (robustness of the tests) | Risk of flaky results at the threshold | ±1 LSB ADC noise can move an averaged reading across a boundary by 0.1 °C | Exact-boundary tests and the STATUS test use a noise-free ADC; other threshold tests keep at least 0.5 °C of margin |
| D5 (code quality) | Extra warnings with -Wconversion and a non-literal printf format | Implicit long-to-int32 conversion and a ternary used as a format string | Explicit cast; format changed to "%s" |

## Lessons learned

- A passing test suite proves little until it has been shown to fail: mutation testing found a gap that normal testing did not.

- Timing tests need an exact measurement mechanism (here a timestamped edge recorder) rather than coarse sleeps.

- Deterministic noise (seeded generator) keeps realistic inputs reproducible.

- Making the interface strict (range and trailing-character checks) removed most edge-case bugs before testing started.

# 10  Quality assurance

## 10.1  Strict compilation

The file compiles with no warnings and no errors using a deliberately strict flag set (-Wall -Wextra -Wpedantic -Wconversion -Wshadow -Wcast-qual -Wstrict-prototypes -Werror).

$ gcc -std=c99 -Wall -Wextra -Wpedantic -Wconversion -Wshadow -Wcast-qual -Wstrict-prototypes -Werror -O2 led_controller.c -o ledctl

(no warnings, no errors)

binary size: 42960 bytes

## 10.2  Sanitizers

The test suite and the demo were also run under AddressSanitizer (memory errors) and UndefinedBehaviorSanitizer (overflow, invalid shifts, misaligned access) with -fno-sanitize-recover=all, so any finding would have aborted the run. Both runs completed cleanly.

$ gcc -std=c99 -Wall -Wextra -Wpedantic -g -O1 -fsanitize=address,undefined led_controller.c -o ledctl_san

$ ./ledctl_san --selftest | tail -3

==============================================

RESULT: 97 checks passed, 0 failed -> ALL PASS

exit code: 0

$ ./ledctl_san --demo > /dev/null

exit code: 0 (no sanitizer reports)

## 10.3  Mutation testing

To check that the tests can actually detect bugs, twelve realistic faults were injected one at a time into a copy of the source (for example an off-by-one in the blink half-period, no hysteresis, no rounding in the ADC conversion, a longer watchdog timeout, a missing buffer-full check). The suite must fail for each mutant. All 12 mutants were detected (the first run detected 11; the gap was closed as described in D3).

| ID | Injected bug | Failed checks | First failing check |
| --- | --- | --- | --- |
| M1 | Blink half-period off by one | 1 | ON time is exactly 100 ms in each cycle (50 % duty) |
| M2 | Alert hysteresis removed | 3 | 34.0 degC is inside the hysteresis band: alert stays on |
| M3 | Ring buffer full check removed | 5 | 37 bytes dropped and counted (63 fit in the ring) |
| M4 | ADC rounding removed | 4 | exactly 35.0 degC raises the alert (>= comparison) |
| M5 | Watchdog timeout too long (5 s) | 5 | watchdog reset happened and was reported |
| M6 | Alert does not override LED mode | 1 | LED forced to 100 ms blink during alert (user mode was OFF) |
| M7 | Moving-average window reduced to 1 | 2 | glitch is visible but divided by the window (peak about 29.4 degC) |
| M8 | TAB handling bug re-introduced | 1 | tab used as separator accepted |
| M9 | BRIGHT upper limit 101 instead of 100 | 1 | BRIGHT 101 rejected, value unchanged |
| M10 | Over-long line not flagged | 1 | over-long line rejected with ERR LINE TOO LONG |
| M11 | Threshold compared with > instead of >= | 2 | exactly 35.0 degC raises the alert (>= comparison) |
| M12 | Breathe peak not scaled by brightness | 1 | BRIGHT 50 scales the breathe peak to 50 % |

## 10.4  Code metrics

| Metric | Value |
| --- | --- |
| Total lines in led_controller.c | 1266 (about 1004 code lines, 169 comment-only lines, 93 blank) |
| Firmware part (Sections A to D) | about 430 lines |
| Simulator (Section E) | about 190 lines |
| Test suite (Section F) and demo/interactive (Section G) | about 560 lines |
| Dynamic memory / floating point in firmware | none / none |
| Compiler warnings with strict flags | 0 |

# 11  Limitations and future work

- The simulator is millisecond-granular and cooperative; real pre-emption races, clock-tree effects and analog behaviour can only be checked on hardware.

- The ADC model is an ideal LM35 with ±1 LSB noise; a real sensor needs calibration and may need a hardware low-pass filter.

- Transmit is polled; a real port could use a transmit ring buffer with the TXE interrupt or DMA.

- Next steps: port the hal layer to an STM32 board (for example with STM32CubeIDE), replace the polled main loop with __WFI() low-power sleep between interrupts, and add DMA for UART receive.

- Possible feature extensions: persistent configuration in flash, additional sensors on I2C, a binary protocol with CRC.

# 12  Development process

The work followed an incremental process in which every phase produced a checkable result. The table lists the phases and the evidence each one left behind in this report and in the source file.

| Phase | Main activities | Evidence |
| --- | --- | --- |
| 1 Planning and requirements | Selecting the application, writing requirements F1 to F9 and N1 to N6, listing test ideas | Section 2, Table 1 |
| 2 Architecture and design | ISR / main-loop split, ring-buffer design, command grammar, timing budget, HAL definition | Section 3, Figure 1, Table 2 |
| 3 Hardware model | HAL functions, simulator, millisecond time stepping, fault-injection hooks, LED edge recorder | Section 5, Table 5 |
| 4 Firmware core | SysTick and UART ISRs, ring buffer, LED engine, tick scheduling | Section 4.1 to 4.4 |
| 5 Features | Command parser and validation, sensor pipeline, alert with hysteresis, watchdog and recovery | Section 4.2 to 4.7 |
| 6 Test suite | 16 test cases, helper functions, demo scenario, interactive mode | Section 7, Table 6; Appendix A |
| 7 Debugging and quality checks | Fixing defects D1 to D5, strict warnings, sanitizers, mutation testing | Section 9 and 10, Tables 8 and 9 |
| 8 Documentation | README and inline comments, plots from the simulation trace, this report | Source file header; Figures 1 to 3 |

# 13  Conclusion

LED-CTRL meets all stated requirements. The single C file contains interrupt-driven firmware, a software model of the board and an automated suite of 16 test cases with 97 checks, all passing. Timing was verified to the millisecond, temperature accuracy to about 0.1 °C, and the firmware survives overflow, malformed input, sensor glitches and a hung main loop. Strict compilation, sanitizers and mutation testing support the quality of both the firmware and the tests. The project demonstrates the full cycle asked for in the task: planning, coding, simulation, testing with feedback, debugging and documentation.

# Appendix A  Complete self-test output

$ ./ledctl --selftest

LED-CTRL firmware self-test (simulated target)

==============================================

TC-01  Boot banner, initial state

[PASS] boot banner shows firmware version

[PASS] reset cause POWER_ON, reset counter 0

[PASS] prompt hint printed

[PASS] LED is off after boot

[PASS] default mode OFF, brightness 100 %

TC-02  SysTick time base accuracy and tick servicing

[PASS] 10 000 SysTick interrupts -> uptime exactly 10 000 ms

[PASS] main loop never fell behind (0 tick overruns)

[PASS] no unserviced ticks left

TC-03  Basic UART commands, case-insensitivity

[PASS] HELP lists the commands

[PASS] VER returns version

[PASS] LED ON replies OK and drives LED to 100 %

[PASS] lower-case 'led off' accepted, LED off

TC-04  BRIGHT command and argument validation

[PASS] BRIGHT 40 -> LED level 40 %

[PASS] BRIGHT 0 accepted (lower bound)

[PASS] BRIGHT 100 accepted (upper bound)

[PASS] BRIGHT 101 rejected, value unchanged

[PASS] BRIGHT -1 rejected

[PASS] BRIGHT abc -> ERR BAD ARG

[PASS] BRIGHT 5x (trailing junk) -> ERR BAD ARG

[PASS] BRIGHT without value -> ERR MISSING ARG

TC-05  Blink timing (timer interrupt accuracy)

[PASS] LED BLINK 200 accepted

[PASS] at least 10 blink cycles in 2.2 s

[PASS] every rising-edge interval is exactly 200 ms

[PASS] ON time is exactly 100 ms in each cycle (50 % duty)

[PASS] period 49 ms rejected (below minimum)

[PASS] period 5001 ms rejected (above maximum)

[PASS] LED BLINK without argument uses default 500 ms

[PASS] minimum period 50 ms generated correctly

TC-06  Breathe (triangle PWM fade)

[PASS] peak brightness reaches 100 %

[PASS] fade returns to 0 %

[PASS] peak occurs half-way through the 2 s cycle

[PASS] level rises monotonically to the peak

[PASS] level falls monotonically after the peak

[PASS] BRIGHT 50 scales the breathe peak to 50 %

TC-07  Input robustness: unknown, blank, tabs, backspace, too many args

[PASS] unknown command -> ERR UNKNOWN CMD

[PASS] blank lines produce no reply

[PASS] tab used as separator accepted

[PASS] extra spaces ignored

[PASS] backspace editing works (LEDD<BS> ON -> LED ON)

[PASS] LED BLINK 100 200 (extra argument) -> ERR TOO MANY ARGS

[PASS] four tokens -> ERR TOO MANY ARGS

[PASS] LED without sub-command -> ERR MISSING ARG

[PASS] LED SPIN -> ERR BAD ARG

TC-08  UART overflow and over-long line handling

[PASS] 37 bytes dropped and counted (63 fit in the ring)

[PASS] over-long line rejected with ERR LINE TOO LONG

[PASS] system fully usable after the overflow

[PASS] STATUS reports rx overflow counter 37

[PASS] a line of exactly 40 characters is accepted

TC-09  Temperature measurement accuracy (ADC + filter)

[PASS] true 0.0 degC -> reported within +-0.2 degC (got 0.1)

[PASS] true 25.0 degC -> reported within +-0.2 degC (got 25.0)

[PASS] true 37.5 degC -> reported within +-0.2 degC (got 37.5)

[PASS] true 50.0 degC -> reported within +-0.2 degC (got 49.9)

[PASS] true 99.0 degC -> reported within +-0.2 degC (got 99.1)

TC-10  Moving average suppresses a single-sample glitch

[PASS] glitch is visible but divided by the window (peak about 29.4 degC)

[PASS] no false alert from a one-sample 60 degC glitch

TC-11  Over-temperature alert, LED override and hysteresis

[PASS] no alert at 25.0 degC

[PASS] ALERT ON raised within 1 s of reaching 40 degC

[PASS] LED forced to 100 ms blink during alert (user mode was OFF)

[PASS] 34.0 degC is inside the hysteresis band: alert stays on

[PASS] ALERT OFF once temperature drops below 33.0 degC

[PASS] LED returns to the user mode (OFF) after the alert

[PASS] 34.5 degC (just below 35.0) does not trigger

[PASS] 34.9 degC (noise-free): no alert

[PASS] exactly 35.0 degC raises the alert (>= comparison)

[PASS] 33.1 degC is still inside the hysteresis band

[PASS] exactly 33.0 degC clears the alert (<= comparison)

TC-12  THRESH command

[PASS] 30.0 degC, default threshold 35: no alert

[PASS] THRESH 28 accepted

[PASS] alert raised after threshold lowered to 28 degC

[PASS] THRESH 9 rejected, old value kept

[PASS] THRESH 101 rejected

[PASS] THRESH hot rejected

[PASS] THRESH without value -> ERR MISSING ARG

TC-13  STATUS report and TEMP before first sample

[PASS] TEMP before the first sample -> ERR NO DATA

[PASS] STATUS shows mode and period

[PASS] STATUS shows brightness and threshold

[PASS] STATUS shows temperature and alert flag

[PASS] STATUS shows health counters

TC-14  Watchdog recovers a hung main loop

[PASS] HANG command stops the main loop

[PASS] no reset before the 2 s timeout

[PASS] reset counter still 0

[PASS] hung firmware ignores UART commands

[PASS] watchdog reset happened and was reported

[PASS] reset occurred about 2000 ms after the hang

[PASS] firmware restarted in its default state

[PASS] reset counter persisted in the backup register and is reported

[PASS] console works again after recovery

TC-15  Ring buffer unit test (wrap-around, full, empty)

[PASS] empty buffer: pop fails

[PASS] 500 bytes pushed/popped in order across many wrap-arounds

[PASS] pushing 74 bytes into a 64-slot ring drops exactly 11 (one slot is the full marker)

[PASS] 63 oldest bytes are preserved intact

TC-16  ADC conversion unit test

[PASS] code 0 -> 0.0 degC

[PASS] code 310 -> 25.0 degC

[PASS] full scale 4095 -> 330.0 degC (3.3 V at 10 mV/degC)

[PASS] code 745 -> 60.0 degC

[PASS] formatter prints negative values (-3.5)

[PASS] formatter prints 0.5

==============================================

RESULT: 97 checks passed, 0 failed -> ALL PASS

exit code: 0

# Appendix B  Demo and interactive session logs

## B.1  Demo scenario (./ledctl --demo)

$ ./ledctl --demo --csv demo_trace.csv

--- DEMO: scripted scenario on the simulated board ---

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[    500 ms] RX> HELP

[    500 ms] TX> COMMANDS: HELP VER LED ON|OFF|BLINK [ms]|BREATHE BRIGHT <0-100> TEMP THRESH <10-100> STATUS HANG

[    502 ms] RX> LED BLINK 400

[    503 ms] TX> OK LED BLINK 400

[   2505 ms] RX> LED BREATHE

[   2506 ms] TX> OK LED BREATHE

[   5508 ms] RX> BRIGHT 60

[   5509 ms] TX> OK BRIGHT 60

[   6511 ms] RX> LED ON

[   6512 ms] TX> OK LED ON

[   7014 ms] RX> LED OFF

[   7015 ms] TX> OK LED OFF

[   7517 ms] RX> TEMP

[   7517 ms] TX> OK TEMP 25.0C

[   7519 ms] RX> LED BLINK 1000

[   7521 ms] TX> OK LED BLINK 1000

[   7523 ms] RX> BRIGHT 200

[   7524 ms] TX> ERR RANGE 0-100

[   7526 ms] RX> FLY

[   7526 ms] TX> ERR UNKNOWN CMD

[   7528 ms] RX> STATUS

[   7529 ms] TX> STATUS mode=BLINK period=1000 bright=60 led=60 temp=25.0C thresh=35C alert=0 up=7529ms rxovf=0 tickovr=0 wdg_resets=0

--- temperature rises 25 -> 40 degC (threshold 35) ---

[  11800 ms] TX> ALERT ON temp=35.0C thresh=35C

[  14531 ms] RX> STATUS

[  14532 ms] TX> STATUS mode=BLINK period=1000 bright=60 led=60 temp=40.0C thresh=35C alert=1 up=14532ms rxovf=0 tickovr=0 wdg_resets=0

--- temperature falls 40 -> 28 degC ---

[  17700 ms] TX> ALERT OFF temp=32.8C thresh=35C

[  20334 ms] RX> STATUS

[  20334 ms] TX> STATUS mode=BLINK period=1000 bright=60 led=0 temp=28.0C thresh=35C alert=0 up=20334ms rxovf=0 tickovr=0 wdg_resets=0

--- watchdog demonstration ---

[  20336 ms] RX> HANG

[  20337 ms] TX> OK HANG main loop stopped, watchdog will reset in 2000 ms

[  22337 ms] TX> BOOT LED-CTRL v1.0.0 reset=WATCHDOG resets=1

[  22337 ms] TX> Type HELP for commands.

[  22939 ms] RX> STATUS

[  22939 ms] TX> STATUS mode=OFF period=500 bright=100 led=0 temp=28.0C thresh=35C alert=0 up=603ms rxovf=0 tickovr=0 wdg_resets=1

--- end of demo (simulated time 22942 ms) ---

## B.2  Test run with UART traffic (excerpt of --selftest --verbose)

$ ./ledctl --selftest --verbose   (excerpt: UART traffic during tests)

LED-CTRL firmware self-test (simulated target)

==============================================

TC-01  Boot banner, initial state

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[PASS] boot banner shows firmware version

[PASS] reset cause POWER_ON, reset counter 0

[PASS] prompt hint printed

[PASS] LED is off after boot

[PASS] default mode OFF, brightness 100 %

TC-02  SysTick time base accuracy and tick servicing

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[PASS] 10 000 SysTick interrupts -> uptime exactly 10 000 ms

[PASS] main loop never fell behind (0 tick overruns)

[PASS] no unserviced ticks left

TC-03  Basic UART commands, case-insensitivity

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[      0 ms] RX> HELP

[      0 ms] TX> COMMANDS: HELP VER LED ON|OFF|BLINK [ms]|BREATHE BRIGHT <0-100> TEMP THRESH <10-100> STATUS HANG

[PASS] HELP lists the commands

[      2 ms] RX> VER

[      2 ms] TX> OK VER 1.0.0

[PASS] VER returns version

[      4 ms] RX> LED ON

[      5 ms] TX> OK LED ON

[PASS] LED ON replies OK and drives LED to 100 %

[     12 ms] RX> led off

[     13 ms] TX> OK LED OFF

[PASS] lower-case 'led off' accepted, LED off

TC-04  BRIGHT command and argument validation

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[      0 ms] RX> LED ON

[      0 ms] TX> OK LED ON

[      2 ms] RX> BRIGHT 40

[      3 ms] TX> OK BRIGHT 40

[PASS] BRIGHT 40 -> LED level 40 %

[     10 ms] RX> BRIGHT 0

[     11 ms] TX> OK BRIGHT 0

[PASS] BRIGHT 0 accepted (lower bound)

[     16 ms] RX> BRIGHT 100

[     17 ms] TX> OK BRIGHT 100

[PASS] BRIGHT 100 accepted (upper bound)

[     22 ms] RX> BRIGHT 101

[     23 ms] TX> ERR RANGE 0-100

[PASS] BRIGHT 101 rejected, value unchanged

[     25 ms] RX> BRIGHT -1

[     26 ms] TX> ERR RANGE 0-100

[PASS] BRIGHT -1 rejected

[     28 ms] RX> BRIGHT abc

[     29 ms] TX> ERR BAD ARG

[PASS] BRIGHT abc -> ERR BAD ARG

[     31 ms] RX> BRIGHT 5x

[     32 ms] TX> ERR BAD ARG

[PASS] BRIGHT 5x (trailing junk) -> ERR BAD ARG

[     34 ms] RX> BRIGHT

[     35 ms] TX> ERR MISSING ARG

[PASS] BRIGHT without value -> ERR MISSING ARG

TC-05  Blink timing (timer interrupt accuracy)

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[      0 ms] RX> LED BLINK 200

[      1 ms] TX> OK LED BLINK 200

[PASS] LED BLINK 200 accepted

[PASS] at least 10 blink cycles in 2.2 s

[PASS] every rising-edge interval is exactly 200 ms

[PASS] ON time is exactly 100 ms in each cycle (50 % duty)

[   2203 ms] RX> LED BLINK 49

[   2204 ms] TX> ERR RANGE 50-5000

[PASS] period 49 ms rejected (below minimum)

[   2206 ms] RX> LED BLINK 5001

[   2207 ms] TX> ERR RANGE 50-5000

[PASS] period 5001 ms rejected (above maximum)

[   2209 ms] RX> LED BLINK

[   2210 ms] TX> OK LED BLINK 500

[PASS] LED BLINK without argument uses default 500 ms

[   2212 ms] RX> LED BLINK 50

[   2213 ms] TX> OK LED BLINK 50

[PASS] minimum period 50 ms generated correctly

TC-06  Breathe (triangle PWM fade)

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[      0 ms] RX> LED BREATHE

[      0 ms] TX> OK LED BREATHE

[PASS] peak brightness reaches 100 %

[PASS] fade returns to 0 %

[PASS] peak occurs half-way through the 2 s cycle

[PASS] level rises monotonically to the peak

[PASS] level falls monotonically after the peak

[   2003 ms] RX> BRIGHT 50

[   2003 ms] TX> OK BRIGHT 50

[PASS] BRIGHT 50 scales the breathe peak to 50 %

TC-07  Input robustness: unknown, blank, tabs, backspace, too many args

[      0 ms] TX> BOOT LED-CTRL v1.0.0 reset=POWER_ON resets=0

[      0 ms] TX> Type HELP for commands.

[      0 ms] RX> FOO

[      0 ms] TX> ERR UNKNOWN CMD

[PASS] unknown command -> ERR UNKNOWN CMD

[PASS] blank lines produce no reply

[      7 ms] RX> LED    OFF

[      8 ms] TX> OK LED OFF

[PASS] tab used as separator accepted

[     10 ms] RX>    led    on

[     11 ms] TX> OK LED ON

[PASS] extra spaces ignored

[     14 ms] TX> OK LED ON

[PASS] backspace editing works (LEDD<BS> ON -> LED ON)

[     17 ms] RX> LED BLINK 100 200

[     19 ms] TX> ERR TOO MANY ARGS

[PASS] LED BLINK 100 200 (extra argument) -> ERR TOO MANY ARGS

[     21 ms] RX> LED A B C D

# Appendix C  Complete source code (led_controller.c)

Line numbers are shown for reference. The same file is submitted separately as led_controller.c.

1  /*

2   * =============================================================================

3   *  LED-CTRL : Timer-interrupt LED controller with UART command console,

4   *             temperature monitoring and watchdog  (simulated embedded target)

5   * =============================================================================

6   *  File     : led_controller.c   (single-file project: firmware + simulator + tests)

7   *  Version  : 1.0.0

8   *  Author   : Saurav Pardeshi   (Embedded Systems Internship - firmware task)

9   *  Language : C99, no dynamic memory, no floating point in the firmware part

10   *

11   * -----------------------------------------------------------------------------

12   *  README

13   * -----------------------------------------------------------------------------

14   *  1. WHAT THIS PROJECT DOES

15   *     Firmware for a small Cortex-M style board with

16   *        - one LED on a PWM-capable pin (brightness 0..100 %),

17   *        - a UART console (115200 8N1) that accepts text commands,

18   *        - an LM35-type analog temperature sensor on a 12-bit ADC,

19   *        - an independent watchdog (2 s timeout).

20   *     The firmware is interrupt driven:

21   *        SysTick_Handler()    1 kHz timer interrupt  -> time base, sets a tick flag

22   *        USART2_IRQHandler()  RX interrupt           -> pushes bytes in a ring buffer

23   *     and a cooperative main loop (app_poll) does all the real work:

24   *        - LED engine: OFF / ON / BLINK <period> / BREATHE (triangle fade)

25   *        - 100 ms temperature sampling, 8-sample moving average,

26   *          over-temperature ALERT with hysteresis (alert overrides the LED mode

27   *          with a fast 100 ms blink)

28   *        - command parser with full input validation and error replies

29   *        - watchdog supervision (kick only while the main loop is alive)

30   *

31   *  2. HOW THE "SIMULATED EMBEDDED SYSTEM" WORKS

32   *     No hardware is needed. Section E of this file is a software model of the

33   *     board (LED pin, UART data register, ADC, watchdog, backup register). It

34   *     calls the same ISR functions a real MCU would call, one millisecond at a

35   *     time. Every hardware access in the firmware goes through the small hal_*

36   *     layer, so porting to a real STM32 means re-implementing ~10 functions:

37   *

38   *        hal_led_set_level()   -> __HAL_TIM_SET_COMPARE(&htim3, TIM_CHANNEL_1, ..)

39   *        hal_uart_putc()       -> HAL_UART_Transmit() / write USART2->TDR

40   *        hal_uart_read_rx()    -> read USART2->RDR  (inside the RXNE interrupt)

41   *        hal_adc_read()        -> HAL_ADC_GetValue()

42   *        hal_wdg_kick()        -> HAL_IWDG_Refresh()

43   *        hal_reset_cause()     -> RCC->CSR IWDGRSTF flag

44   *        hal_persist_get/set() -> RTC backup register

45   *        hal_irq_disable/enable-> __disable_irq() / __enable_irq()

46   *

47   *  3. HOW TO COMPILE

48   *        gcc -std=c99 -Wall -Wextra -Wpedantic -Werror -O2 led_controller.c -o ledctl

49   *     Optional (recommended) sanitizer build used during development:

50   *        gcc -std=c99 -Wall -Wextra -Wpedantic -g -O1 \

51   *            -fsanitize=address,undefined led_controller.c -o ledctl_san

52   *     Works with gcc or clang on Linux, macOS and Windows (MinGW / WSL).

53   *

54   *  4. HOW TO RUN

55   *        ./ledctl --selftest            run the automated test suite (exit code 0 = all pass)

56   *        ./ledctl --demo                scripted scenario with a time-stamped UART log

57   *        ./ledctl --demo --csv out.csv  same, plus a 1 ms CSV trace (t, led, temp, alert)

58   *        ./ledctl --interactive         type UART commands yourself (see below)

59   *        ./ledctl --selftest --verbose  also print every UART line during the tests

60   *

61   *     Interactive mode: every line you type is sent to the simulated UART.

62   *     Extra simulator directives (start with '@'):

63   *        @wait <ms>       advance simulated time          (e.g. @wait 2000)

64   *        @temp <degC>     set the real temperature        (e.g. @temp 38.5)

65   *        @quit            leave

66   *

67   *  5. UART COMMAND REFERENCE (case-insensitive, end with Enter / CR / LF)

68   *        HELP                  list commands

69   *        VER                   firmware version

70   *        LED ON | LED OFF      steady on at current brightness / off

71   *        LED BLINK [ms]        blink with full period 50..5000 ms (default 500)

72   *        LED BREATHE           2 s triangle fade up/down

73   *        BRIGHT <0..100>       LED brightness in percent

74   *        TEMP                  filtered temperature in degC

75   *        THRESH <10..100>      over-temperature alert threshold in degC

76   *        STATUS                one-line status report

77   *        HANG                  DEBUG: stop the main loop to demonstrate the watchdog

78   *     Replies start with "OK" or "ERR". Asynchronous events: "ALERT ON/OFF ...".

79   *

80   *  6. KEY FUNCTIONS

81   *        SysTick_Handler      1 ms ISR, increments time base, raises tick flag

82   *        USART2_IRQHandler    RX ISR, lock-free single-producer ring buffer

83   *        app_poll             main-loop step: service ticks, UART, watchdog

84   *        on_tick_1ms          1 ms work: LED engine + sample scheduler

85   *        led_engine_step      generates OFF/ON/BLINK/BREATHE output

86   *        sensor_sample        ADC read, convert, moving average, alert logic

87   *        uart_rx_service      assembles lines (backspace, overflow handling)

88   *        process_line         tokenise, validate and execute commands

89   *        fw_boot              (re)initialise everything, print boot banner

90   *

91   *  7. TEST SUMMARY (details in the accompanying report)

92   *        16 test cases / 97 individual checks, run with --selftest (all pass).

93   *        Clean under -Wall -Wextra -Wpedantic -Wconversion -Wshadow -Werror and under

94   *        AddressSanitizer + UndefinedBehaviorSanitizer. Mutation testing: 12 of 12

95   *        deliberately injected bugs are detected by the suite.

96   * =============================================================================

97   */

98

99  #include <ctype.h>

100  #include <errno.h>

101  #include <stdarg.h>

102  #include <stdbool.h>

103  #include <stdint.h>

104  #include <stdio.h>

105  #include <stdlib.h>

106  #include <string.h>

107

108  /* =============================================================================

109   *  SECTION A : CONFIGURATION CONSTANTS

110   * ============================================================================= */

111  #define FW_VERSION          "1.0.0"

112

113  #define UART_RX_BUF_SIZE    64u      /* ring buffer size, MUST be a power of two      */

114  #define CMD_MAX_LEN         40u      /* longest accepted command line (characters)   */

115  #define MAX_TOKENS          3        /* command word + up to two arguments           */

116

117  #define BLINK_MIN_MS        50       /* allowed full blink period                    */

118  #define BLINK_MAX_MS        5000

119  #define BLINK_DEFAULT_MS    500

120  #define BREATHE_PERIOD_MS   2000     /* full fade-up + fade-down cycle               */

121  #define ALERT_BLINK_MS      100      /* blink period used while the alert is active  */

122

123  #define ADC_MAX             4095     /* 12-bit converter                             */

124  #define VREF_MV             3300     /* ADC reference voltage                        */

125  #define TEMP_SAMPLE_MS      100      /* sensor sampling interval                     */

126  #define AVG_WINDOW          8        /* moving-average length (samples)              */

127  #define THRESH_DEFAULT_C    35       /* default alert threshold in degC              */

128  #define THRESH_MIN_C        10

129  #define THRESH_MAX_C        100

130  #define HYST_C10            20       /* alert hysteresis: 2.0 degC (tenths of degC)  */

131

132  #define WDG_TIMEOUT_MS      2000     /* independent watchdog timeout                 */

133

134  /* Temperatures are stored as integers in TENTHS of a degree Celsius ("c10").

135   * LM35: 10 mV per degC  ->  1 mV == 0.1 degC  ->  c10 value == millivolts.     */

136

137  /* =============================================================================

138   *  SECTION B : TYPES

139   * ============================================================================= */

140  typedef enum { LED_OFF = 0, LED_ON, LED_BLINK, LED_BREATHE } led_mode_t;

141  typedef enum { RESET_POWER_ON = 0, RESET_WATCHDOG } reset_cause_t;

142

143  static const char *const k_mode_name[] = { "OFF", "ON", "BLINK", "BREATHE" };

144  static const char *const k_cause_name[] = { "POWER_ON", "WATCHDOG" };

145

146  /* =============================================================================

147   *  SECTION C : HARDWARE ABSTRACTION LAYER (interface the firmware relies on)

148   *  Implemented by the simulator in Section E; replace for a real board.

149   * ============================================================================= */

150  static void          hal_led_set_level(uint8_t percent);

151  static void          hal_uart_putc(uint8_t c);

152  static uint8_t       hal_uart_read_rx(void);

153  static uint16_t      hal_adc_read(void);

154  static void          hal_wdg_kick(void);

155  static reset_cause_t hal_reset_cause(void);

156  static uint8_t       hal_persist_get(void);

157  static void          hal_persist_set(uint8_t v);

158  static void          hal_irq_disable(void);

159  static void          hal_irq_enable(void);

160

161  #define CRIT_ENTER()  hal_irq_disable()

162  #define CRIT_EXIT()   hal_irq_enable()

163

164  /* =============================================================================

165   *  SECTION D : FIRMWARE (the part that would run on the microcontroller)

166   * ============================================================================= */

167

168  /* ---- state shared between ISRs and main loop (volatile) ---- */

169  static volatile uint32_t g_ms;              /* milliseconds since boot (SysTick)    */

170  static volatile uint32_t g_ticks_pending;   /* ticks not yet serviced by main loop  */

171  static volatile uint8_t  rx_head;           /* written by ISR only                  */

172  static volatile uint8_t  rx_tail;           /* written by main loop only            */

173  static volatile uint32_t rx_overflow;       /* bytes dropped because buffer was full */

174  static uint8_t           rx_buf[UART_RX_BUF_SIZE];

175

176  /* ---- main-loop-only state ---- */

177  static uint32_t tick_overruns;              /* times the main loop fell >1 tick behind */

178  static led_mode_t user_mode;

179  static uint16_t blink_ms;

180  static uint8_t  brightness;                 /* 0..100 percent                       */

181  static uint16_t phase_ms;                   /* position inside blink/breathe cycle  */

182  static uint8_t  level_applied;              /* last value written to the LED        */

183

184  static int16_t  t_win[AVG_WINDOW];          /* moving-average window (c10)          */

185  static uint8_t  t_idx, t_cnt;

186  static int32_t  t_avg_c10;

187  static uint8_t  sample_countdown;

188  static int32_t  thresh_c;

189  static bool     alert_active;

190

191  static char     line_buf[CMD_MAX_LEN + 1u];

192  static uint8_t  line_len;

193  static bool     line_overflow;

194  static bool     g_hung;                     /* set by HANG: main loop stops          */

195

196  /* ----------------------------------------------------------------------------

197   *  Interrupt service routines

198   * -------------------------------------------------------------------------- */

199

200  /* 1 kHz time base. Kept extremely short: only counts and raises a flag. */

201  void SysTick_Handler(void)

202  {

203      g_ms++;

204      g_ticks_pending++;

205  }

206

207  /* UART receive interrupt: move the received byte into the ring buffer.

208   * head is only written here, tail only by the main loop -> no lock needed. */

209  void USART2_IRQHandler(void)

210  {

211      uint8_t b = hal_uart_read_rx();

212      uint8_t next = (uint8_t)((rx_head + 1u) & (UART_RX_BUF_SIZE - 1u));

213      if (next == rx_tail) {          /* buffer full: drop byte, count the loss */

214          rx_overflow++;

215          return;

216      }

217      rx_buf[rx_head] = b;

218      rx_head = next;

219  }

220

221  /* ----------------------------------------------------------------------------

222   *  UART helpers

223   * -------------------------------------------------------------------------- */

224  static bool rx_pop(uint8_t *out)

225  {

226      if (rx_tail == rx_head) {

227          return false;

228      }

229      *out = rx_buf[rx_tail];

230      rx_tail = (uint8_t)((rx_tail + 1u) & (UART_RX_BUF_SIZE - 1u));

231      return true;

232  }

233

234  /* printf-style transmit (blocking, polled). Real board: could use a TX ring + TXE IRQ. */

235  static void uart_printf(const char *fmt, ...)

236  {

237      char buf[200];

238      va_list ap;

239      va_start(ap, fmt);

240      (void)vsnprintf(buf, sizeof buf, fmt, ap);

241      va_end(ap);

242      for (const char *p = buf; *p != '\0'; p++) {

243          hal_uart_putc((uint8_t)*p);

244      }

245  }

246

247  /* Format a tenths-of-degree value as "-12.3" without using floating point. */

248  static const char *fmt_c10(int32_t v, char *out, size_t n)

249  {

250      int32_t a = (v < 0) ? -v : v;

251      (void)snprintf(out, n, "%s%ld.%ld", (v < 0) ? "-" : "", (long)(a / 10), (long)(a % 10));

252      return out;

253  }

254

255  /* ----------------------------------------------------------------------------

256   *  LED engine (called once per millisecond tick)

257   * -------------------------------------------------------------------------- */

258  static void led_restart_phase(void)

259  {

260      phase_ms = 0u;

261  }

262

263  static void led_engine_step(void)

264  {

265      led_mode_t mode   = alert_active ? LED_BLINK : user_mode;

266      uint16_t   period = alert_active ? (uint16_t)ALERT_BLINK_MS : blink_ms;

267      uint8_t    level  = 0u;

268

269      switch (mode) {

270      case LED_OFF:

271          level = 0u;

272          break;

273      case LED_ON:

274          level = brightness;

275          break;

276      case LED_BLINK: {

277          /* first half of the period ON, second half OFF */

278          level = (phase_ms < (uint16_t)(period / 2u)) ? brightness : 0u;

279          if (++phase_ms >= period) {

280              phase_ms = 0u;

281          }

282          break;

283      }

284      case LED_BREATHE: {

285          /* triangle wave 0..1000..0 over BREATHE_PERIOD_MS, scaled by brightness */

286          uint32_t half = BREATHE_PERIOD_MS / 2u;

287          uint32_t tri  = (phase_ms < half) ? phase_ms : (BREATHE_PERIOD_MS - phase_ms);

288          level = (uint8_t)((tri * brightness) / half);

289          if (++phase_ms >= BREATHE_PERIOD_MS) {

290              phase_ms = 0u;

291          }

292          break;

293      }

294      default:

295          level = 0u;

296          break;

297      }

298

299      if (level != level_applied) {   /* only touch the hardware when needed */

300          level_applied = level;

301          hal_led_set_level(level);

302      }

303  }

304

305  /* ----------------------------------------------------------------------------

306   *  Temperature sensing

307   * -------------------------------------------------------------------------- */

308

309  /* Raw 12-bit ADC code -> tenths of degC (LM35: 10 mV/degC, so c10 == mV). */

310  static int32_t adc_to_c10(uint16_t raw)

311  {

312      uint32_t mv = ((uint32_t)raw * VREF_MV + (ADC_MAX / 2u)) / ADC_MAX;   /* rounded */

313      return (int32_t)mv;

314  }

315

316  static void alert_set(bool on)

317  {

318      char tb[16];

319      if (on == alert_active) {

320          return;

321      }

322      alert_active = on;

323      led_restart_phase();            /* start the blink pattern cleanly */

324      uart_printf("ALERT %s temp=%sC thresh=%ldC\r\n", on ? "ON" : "OFF",

325                  fmt_c10(t_avg_c10, tb, sizeof tb), (long)thresh_c);

326  }

327

328  /* Take one sample, update the moving average, evaluate the alert with hysteresis. */

329  static void sensor_sample(void)

330  {

331      t_win[t_idx] = (int16_t)adc_to_c10(hal_adc_read());

332      t_idx = (uint8_t)((t_idx + 1u) % AVG_WINDOW);

333      if (t_cnt < AVG_WINDOW) {

334          t_cnt++;

335      }

336

337      int32_t sum = 0;

338      for (uint8_t i = 0; i < t_cnt; i++) {

339          sum += t_win[i];

340      }

341      t_avg_c10 = (sum + (int32_t)(t_cnt / 2u)) / (int32_t)t_cnt;

342

343      if (t_cnt == AVG_WINDOW) {      /* judge only once the window is full */

344          int32_t hi = thresh_c * 10;

345          if (!alert_active && t_avg_c10 >= hi) {

346              alert_set(true);

347          } else if (alert_active && t_avg_c10 <= (hi - HYST_C10)) {

348              alert_set(false);

349          }

350      }

351  }

352

353  /* ----------------------------------------------------------------------------

354   *  Command parser

355   * -------------------------------------------------------------------------- */

356

357  /* Strict integer parse: whole string must be a number inside [lo, hi]. */

358  static bool parse_int(const char *s, long lo, long hi, long *out)

359  {

360      char *end = NULL;

361      errno = 0;

362      long v = strtol(s, &end, 10);

363      if (errno != 0 || end == s || *end != '\0') {

364          return false;               /* not a clean number */

365      }

366      if (v < lo || v > hi) {

367          return false;

368      }

369      *out = v;

370      return true;

371  }

372

373  /* Split a line in place into at most MAX_TOKENS words. Returns count, or -1 if too many. */

374  static int tokenize(char *line, char *tok[], int max)

375  {

376      int n = 0;

377      char *p = line;

378      while (*p != '\0') {

379          while (*p == ' ' || *p == '\t') {

380              *p++ = '\0';

381          }

382          if (*p == '\0') {

383              break;

384          }

385          if (n == max) {

386              return -1;

387          }

388          tok[n++] = p;

389          while (*p != '\0' && *p != ' ' && *p != '\t') {

390              p++;

391          }

392      }

393      return n;

394  }

395

396  static void cmd_status(void)

397  {

398      char tb[16];

399      if (t_cnt > 0u) {

400          (void)fmt_c10(t_avg_c10, tb, sizeof tb);

401      } else {

402          (void)snprintf(tb, sizeof tb, "n/a");

403      }

404      uart_printf("STATUS mode=%s period=%u bright=%u led=%u temp=%sC thresh=%ldC alert=%u "

405                  "up=%lums rxovf=%lu tickovr=%lu wdg_resets=%u\r\n",

406                  k_mode_name[user_mode], (unsigned)blink_ms, (unsigned)brightness,

407                  (unsigned)level_applied, tb, (long)thresh_c, alert_active ? 1u : 0u,

408                  (unsigned long)g_ms, (unsigned long)rx_overflow, (unsigned long)tick_overruns,

409                  (unsigned)hal_persist_get());

410  }

411

412  static void process_line(char *line)

413  {

414      char *tok[MAX_TOKENS];

415      long  v = 0;

416

417      for (char *p = line; *p != '\0'; p++) {                 /* case-insensitive */

418          *p = (char)toupper((unsigned char)*p);

419      }

420      int n = tokenize(line, tok, MAX_TOKENS);

421      if (n < 0) {

422          uart_printf("ERR TOO MANY ARGS\r\n");

423          return;

424      }

425      if (n == 0) {

426          return;                                             /* blank line: ignore */

427      }

428

429      if (strcmp(tok[0], "HELP") == 0) {

430          uart_printf("COMMANDS: HELP VER LED ON|OFF|BLINK [ms]|BREATHE BRIGHT <0-100> "

431                      "TEMP THRESH <10-100> STATUS HANG\r\n");

432      } else if (strcmp(tok[0], "VER") == 0) {

433          uart_printf("OK VER " FW_VERSION "\r\n");

434      } else if (strcmp(tok[0], "LED") == 0) {

435          if (n < 2) {

436              uart_printf("ERR MISSING ARG\r\n");

437          } else if (strcmp(tok[1], "ON") == 0 && n == 2) {

438              user_mode = LED_ON;  led_restart_phase();  uart_printf("OK LED ON\r\n");

439          } else if (strcmp(tok[1], "OFF") == 0 && n == 2) {

440              user_mode = LED_OFF; led_restart_phase();  uart_printf("OK LED OFF\r\n");

441          } else if (strcmp(tok[1], "BREATHE") == 0 && n == 2) {

442              user_mode = LED_BREATHE; led_restart_phase(); uart_printf("OK LED BREATHE\r\n");

443          } else if (strcmp(tok[1], "BLINK") == 0) {

444              long period = BLINK_DEFAULT_MS;

445              if (n == 3 && !parse_int(tok[2], BLINK_MIN_MS, BLINK_MAX_MS, &period)) {

446                  uart_printf("ERR RANGE %d-%d\r\n", BLINK_MIN_MS, BLINK_MAX_MS);

447              } else {

448                  blink_ms = (uint16_t)period;

449                  user_mode = LED_BLINK;

450                  led_restart_phase();

451                  uart_printf("OK LED BLINK %u\r\n", (unsigned)blink_ms);

452              }

453          } else {

454              uart_printf("ERR BAD ARG\r\n");

455          }

456      } else if (strcmp(tok[0], "BRIGHT") == 0) {

457          if (n < 2) {

458              uart_printf("ERR MISSING ARG\r\n");

459          } else if (n > 2) {

460              uart_printf("ERR TOO MANY ARGS\r\n");

461          } else if (!parse_int(tok[1], 0, 100, &v)) {

462              /* distinguish "not a number" from "out of range" for a helpful message */

463              char *end = NULL;

464              (void)strtol(tok[1], &end, 10);

465              uart_printf("%s", (end != tok[1] && *end == '\0') ? "ERR RANGE 0-100\r\n" : "ERR BAD ARG\r\n");

466          } else {

467              brightness = (uint8_t)v;

468              uart_printf("OK BRIGHT %u\r\n", (unsigned)brightness);

469          }

470      } else if (strcmp(tok[0], "TEMP") == 0) {

471          char tb[16];

472          if (t_cnt == 0u) {

473              uart_printf("ERR NO DATA\r\n");

474          } else {

475              uart_printf("OK TEMP %sC\r\n", fmt_c10(t_avg_c10, tb, sizeof tb));

476          }

477      } else if (strcmp(tok[0], "THRESH") == 0) {

478          if (n < 2) {

479              uart_printf("ERR MISSING ARG\r\n");

480          } else if (!parse_int(tok[1], THRESH_MIN_C, THRESH_MAX_C, &v)) {

481              uart_printf("ERR RANGE %d-%d\r\n", THRESH_MIN_C, THRESH_MAX_C);

482          } else {

483              thresh_c = (int32_t)v;

484              uart_printf("OK THRESH %ld\r\n", v);

485          }

486      } else if (strcmp(tok[0], "STATUS") == 0) {

487          cmd_status();

488      } else if (strcmp(tok[0], "HANG") == 0) {

489          uart_printf("OK HANG main loop stopped, watchdog will reset in %d ms\r\n", WDG_TIMEOUT_MS);

490          g_hung = true;

491      } else {

492          uart_printf("ERR UNKNOWN CMD\r\n");

493      }

494  }

495

496  /* Drain the RX ring buffer and assemble lines. Handles backspace and over-long lines. */

497  static void uart_rx_service(void)

498  {

499      uint8_t c;

500      while (rx_pop(&c)) {

501          if (c == '\r' || c == '\n') {

502              if (line_overflow) {

503                  uart_printf("ERR LINE TOO LONG\r\n");

504              } else if (line_len > 0u) {

505                  line_buf[line_len] = '\0';

506                  process_line(line_buf);

507              }

508              line_len = 0u;

509              line_overflow = false;

510          } else if ((c == 0x08u || c == 0x7Fu)) {            /* backspace / DEL */

511              if (line_len > 0u && !line_overflow) {

512                  line_len--;

513              }

514          } else if ((c >= 0x20u && c < 0x7Fu) || c == '\t') {  /* printable ASCII or TAB */

515              if (line_len < CMD_MAX_LEN && !line_overflow) {

516                  line_buf[line_len++] = (char)c;

517              } else {

518                  line_overflow = true;                       /* discard until end of line */

519              }

520          }

521          /* other control characters are ignored */

522      }

523  }

524

525  /* ----------------------------------------------------------------------------

526   *  Scheduler: 1 ms work and the main-loop step

527   * -------------------------------------------------------------------------- */

528  static void on_tick_1ms(void)

529  {

530      led_engine_step();

531      if (sample_countdown > 0u) {

532          sample_countdown--;

533      }

534      if (sample_countdown == 0u) {

535          sample_countdown = (uint8_t)TEMP_SAMPLE_MS;

536          sensor_sample();

537      }

538  }

539

540  /* One pass of the main loop ("superloop"). On real hardware: while(1){ app_poll(); __WFI(); } */

541  static void app_poll(void)

542  {

543      if (g_hung) {

544          return;                     /* simulates a stuck main loop: nothing is serviced */

545      }

546

547      /* 1. service every pending 1 ms tick, remember if we ever lagged behind */

548      for (;;) {

549          uint32_t pending;

550          CRIT_ENTER();

551          pending = g_ticks_pending;

552          if (pending > 0u) {

553              g_ticks_pending--;

554          }

555          CRIT_EXIT();

556          if (pending == 0u) {

557              break;

558          }

559          if (pending > 1u) {

560              tick_overruns++;

561          }

562          on_tick_1ms();

563      }

564

565      /* 2. handle received characters */

566      uart_rx_service();

567

568      /* 3. watchdog: only kick if we got here, i.e. the loop is alive */

569      if (!g_hung) {

570          hal_wdg_kick();

571      }

572  }

573

574  /* Initialise all firmware state; called at power-on and after every reset. */

575  static void fw_boot(reset_cause_t cause)

576  {

577      g_ms = 0u;  g_ticks_pending = 0u;

578      rx_head = 0u;  rx_tail = 0u;  rx_overflow = 0u;

579      tick_overruns = 0u;

580      user_mode = LED_OFF;  blink_ms = BLINK_DEFAULT_MS;  brightness = 100u;

581      phase_ms = 0u;  level_applied = 0u;

582      memset(t_win, 0, sizeof t_win);

583      t_idx = 0u;  t_cnt = 0u;  t_avg_c10 = 0;

584      sample_countdown = (uint8_t)TEMP_SAMPLE_MS;

585      thresh_c = THRESH_DEFAULT_C;

586      alert_active = false;

587      line_len = 0u;  line_overflow = false;  g_hung = false;

588

589      hal_led_set_level(0u);

590      if (cause == RESET_WATCHDOG) {

591          hal_persist_set((uint8_t)(hal_persist_get() + 1u));   /* count watchdog resets */

592      }

593      uart_printf("\r\nBOOT LED-CTRL v" FW_VERSION " reset=%s resets=%u\r\n",

594                  k_cause_name[cause], (unsigned)hal_persist_get());

595      uart_printf("Type HELP for commands.\r\n");

596  }

597

598  /* =============================================================================

599   *  SECTION E : SIMULATED HARDWARE (software model of the board)

600   *  Not part of the firmware: it plays the role of the MCU peripherals.

601   * ============================================================================= */

602  #define TX_CAP      16384u

603  #define TRACE_MAX   1024u

604  #define HIST_MAX    4096u

605

606  typedef struct {

607      uint32_t      t_ms;               /* absolute simulated time (survives resets)   */

608      int32_t       temp_c10;           /* real temperature at the sensor (tenths degC) */

609      int32_t       spike_raw;          /* >= 0: next ADC conversion returns this code  */

610      bool          noise;              /* add +-1 LSB deterministic ADC noise          */

611      uint32_t      lcg;                /* noise generator state                        */

612      uint8_t       led_level;          /* LED pin PWM duty, percent                    */

613      uint8_t       rdr;                /* UART receive data register                   */

614      uint8_t       persist;            /* backup register (survives reset)             */

615      reset_cause_t cause;

616      bool          wdg_on;

617      uint32_t      wdg_cnt;            /* ms since last kick                           */

618      uint32_t      us_acc;             /* sub-millisecond UART byte-time accumulator   */

619      uint32_t      last_reset_t;       /* simulated time of the last watchdog reset    */

620

621      char          tx[TX_CAP];         /* everything the firmware transmitted          */

622      size_t        tx_len;

623      bool          echo;               /* print RX/TX lines with timestamps            */

624      char          eline[256];

625      size_t        eline_len;

626

627      uint32_t      rise_t[TRACE_MAX], fall_t[TRACE_MAX];   /* LED edge timestamps      */

628      uint32_t      rise_n, fall_n;

629      uint8_t       led_min, led_max;   /* min/max LED level since last stats reset     */

630      uint8_t       hist[HIST_MAX];     /* LED level, one sample per ms (optional)      */

631      uint32_t      hist_n;

632      bool          hist_on;

633      FILE         *csv;

634  } sim_t;

635

636  static sim_t sim;

637

638  /* ---- HAL implementation for the simulator ---- */

639  static void hal_led_set_level(uint8_t pct)

640  {

641      if (sim.led_level == 0u && pct > 0u && sim.rise_n < TRACE_MAX) {

642          sim.rise_t[sim.rise_n++] = sim.t_ms;

643      }

644      if (sim.led_level > 0u && pct == 0u && sim.fall_n < TRACE_MAX) {

645          sim.fall_t[sim.fall_n++] = sim.t_ms;

646      }

647      sim.led_level = pct;

648  }

649

650  static void hal_uart_putc(uint8_t c)

651  {

652      if (sim.tx_len < TX_CAP - 1u) {

653          sim.tx[sim.tx_len++] = (char)c;

654          sim.tx[sim.tx_len] = '\0';

655      }

656      if (sim.echo) {                                  /* timestamped line echo */

657          if (c == '\n') {

658              if (sim.eline_len > 0u) {

659                  printf("[%7lu ms] TX> %s\n", (unsigned long)sim.t_ms, sim.eline);

660              }

661              sim.eline_len = 0u;

662              sim.eline[0] = '\0';

663          } else if (c != '\r' && sim.eline_len < sizeof sim.eline - 1u) {

664              sim.eline[sim.eline_len++] = (char)c;

665              sim.eline[sim.eline_len] = '\0';

666          }

667      }

668  }

669

670  static uint8_t hal_uart_read_rx(void)        { return sim.rdr; }

671  static void    hal_wdg_kick(void)            { sim.wdg_cnt = 0u; }

672  static reset_cause_t hal_reset_cause(void)   { return sim.cause; }

673  static uint8_t hal_persist_get(void)         { return sim.persist; }

674  static void    hal_persist_set(uint8_t v)    { sim.persist = v; }

675  static void    hal_irq_disable(void)         { /* single-threaded simulator: nothing to do */ }

676  static void    hal_irq_enable(void)          { }

677

678  static uint16_t hal_adc_read(void)

679  {

680      int32_t  mv  = sim.temp_c10;                 /* LM35: 1 mV == 0.1 degC */

681      int32_t  raw;

682      if (mv < 0) { mv = 0; }

683      raw = (int32_t)((((uint32_t)mv * ADC_MAX) + (VREF_MV / 2u)) / VREF_MV);

684      if (sim.spike_raw >= 0) {                    /* one-shot injected glitch */

685          raw = sim.spike_raw;

686          sim.spike_raw = -1;

687      } else if (sim.noise) {                      /* deterministic +-1 LSB noise */

688          sim.lcg = sim.lcg * 1664525u + 1013904223u;

689          raw += (int32_t)((sim.lcg >> 16) % 3u) - 1;

690      }

691      if (raw < 0) { raw = 0; }

692      if (raw > ADC_MAX) { raw = ADC_MAX; }

693      return (uint16_t)raw;

694  }

695

696  /* ---- simulator control ---- */

697  static void sim_init(void)

698  {

699      FILE *keep_csv = sim.csv;

700      bool  keep_echo = sim.echo;

701      memset(&sim, 0, sizeof sim);

702      sim.csv = keep_csv;

703      sim.echo = keep_echo;

704      sim.temp_c10 = 250;                          /* room temperature 25.0 degC */

705      sim.spike_raw = -1;

706      sim.noise = true;

707      sim.lcg = 12345u;

708      sim.wdg_on = true;

709      sim.cause = RESET_POWER_ON;

710      sim.led_min = 255u;

711  }

712

713  static void sim_tx_clear(void) { sim.tx_len = 0u; sim.tx[0] = '\0'; }

714  static bool sim_tx_has(const char *s) { return strstr(sim.tx, s) != NULL; }

715

716  static void sim_hw_reset(reset_cause_t cause)

717  {

718      sim.cause = cause;

719      sim.wdg_cnt = 0u;

720      sim.last_reset_t = sim.t_ms;

721      sim.led_level = 0u;                          /* pins return to reset state */

722      fw_boot(hal_reset_cause());

723  }

724

725  /* Advance the whole system by exactly one millisecond. */

726  static void sim_step(void)

727  {

728      sim.t_ms++;

729      if (sim.wdg_on) {

730          sim.wdg_cnt++;

731          if (sim.wdg_cnt >= WDG_TIMEOUT_MS) {

732              sim_hw_reset(RESET_WATCHDOG);        /* independent watchdog bites */

733          }

734      }

735      SysTick_Handler();                           /* timer interrupt fires      */

736      app_poll();                                  /* main loop gets CPU time    */

737

738      if (sim.led_level < sim.led_min) { sim.led_min = sim.led_level; }

739      if (sim.led_level > sim.led_max) { sim.led_max = sim.led_level; }

740      if (sim.hist_on && sim.hist_n < HIST_MAX) { sim.hist[sim.hist_n++] = sim.led_level; }

741      if (sim.csv != NULL) {

742          fprintf(sim.csv, "%lu,%u,%ld,%ld,%u\n", (unsigned long)sim.t_ms, (unsigned)sim.led_level,

743                  (long)sim.temp_c10, (long)(t_cnt > 0u ? t_avg_c10 : 0), alert_active ? 1u : 0u);

744      }

745  }

746

747  static void sim_run(uint32_t ms) { while (ms-- > 0u) { sim_step(); } }

748

749  /* Send text over the simulated UART at 115200 baud (about 87 us per byte). */

750  static void sim_uart_send(const char *s)

751  {

752      for (; *s != '\0'; s++) {

753          sim.rdr = (uint8_t)*s;

754          USART2_IRQHandler();                     /* RXNE interrupt              */

755          app_poll();

756          sim.us_acc += 87u;

757          while (sim.us_acc >= 1000u) {

758              sim.us_acc -= 1000u;

759              sim_step();

760          }

761      }

762  }

763

764  /* Inject bytes faster than the main loop can drain them (ISR only, no polling). */

765  static void sim_uart_burst(const char *s)

766  {

767      for (; *s != '\0'; s++) {

768          sim.rdr = (uint8_t)*s;

769          USART2_IRQHandler();

770      }

771  }

772

773  /* Type a command line as a terminal would (text + CR LF) and let the firmware answer. */

774  static void sim_send_line(const char *s)

775  {

776      if (sim.echo) {

777          printf("[%7lu ms] RX> %s\n", (unsigned long)sim.t_ms, s);

778      }

779      sim_uart_send(s);

780      sim_uart_send("\r\n");

781      sim_run(2u);

782  }

783

784  static void sim_edge_reset(void) { sim.rise_n = 0u; sim.fall_n = 0u; }

785  static void sim_stats_reset(void) { sim.led_min = 255u; sim.led_max = 0u; }

786

787  /* =============================================================================

788   *  SECTION F : AUTOMATED TEST SUITE (--selftest)

789   * ============================================================================= */

790  static int g_pass, g_fail;

791

792  #define CHECK(cond, desc)                                                         \

793      do {                                                                          \

794          if (cond) { g_pass++; printf("    [PASS] %s\n", desc); }                  \

795          else      { g_fail++; printf("    [FAIL] %s  (line %d)\n", desc, __LINE__); } \

796      } while (0)

797

798  static void tc_begin(const char *id, const char *title)

799  {

800      printf("\n%s  %s\n", id, title);

801      sim_init();

802      fw_boot(hal_reset_cause());

803  }

804

805  /* Parse the last "OK TEMP xx.x" reply into tenths of degC. */

806  static bool sim_read_temp_reply(int32_t *c10)

807  {

808      const char *p = NULL, *q = sim.tx;

809      int a = 0, b = 0;

810      while ((q = strstr(q, "OK TEMP ")) != NULL) { p = q; q += 8; }

811      if (p == NULL || sscanf(p + 8, "%d.%d", &a, &b) != 2) { return false; }

812      *c10 = a * 10 + b;

813      return true;

814  }

815

816  static bool all_rise_intervals(uint32_t expect, uint32_t skip)

817  {

818      if (sim.rise_n < skip + 3u) { return false; }

819      for (uint32_t i = skip + 1u; i < sim.rise_n; i++) {

820          if (sim.rise_t[i] - sim.rise_t[i - 1u] != expect) { return false; }

821      }

822      return true;

823  }

824

825  static void tc01_boot(void)

826  {

827      tc_begin("TC-01", "Boot banner, initial state");

828      CHECK(sim_tx_has("BOOT LED-CTRL v" FW_VERSION), "boot banner shows firmware version");

829      CHECK(sim_tx_has("reset=POWER_ON resets=0"), "reset cause POWER_ON, reset counter 0");

830      CHECK(sim_tx_has("Type HELP for commands."), "prompt hint printed");

831      CHECK(sim.led_level == 0u, "LED is off after boot");

832      CHECK(user_mode == LED_OFF && brightness == 100u, "default mode OFF, brightness 100 %");

833  }

834

835  static void tc02_timebase(void)

836  {

837      tc_begin("TC-02", "SysTick time base accuracy and tick servicing");

838      sim_run(10000u);

839      CHECK(g_ms == 10000u, "10 000 SysTick interrupts -> uptime exactly 10 000 ms");

840      CHECK(tick_overruns == 0u, "main loop never fell behind (0 tick overruns)");

841      CHECK(g_ticks_pending == 0u, "no unserviced ticks left");

842  }

843

844  static void tc03_basic_commands(void)

845  {

846      tc_begin("TC-03", "Basic UART commands, case-insensitivity");

847      sim_tx_clear();

848      sim_send_line("HELP");

849      CHECK(sim_tx_has("COMMANDS:") && sim_tx_has("LED ON|OFF|BLINK"), "HELP lists the commands");

850      sim_tx_clear();

851      sim_send_line("VER");

852      CHECK(sim_tx_has("OK VER " FW_VERSION), "VER returns version");

853      sim_tx_clear();

854      sim_send_line("LED ON");

855      sim_run(5u);

856      CHECK(sim_tx_has("OK LED ON") && sim.led_level == 100u, "LED ON replies OK and drives LED to 100 %");

857      sim_tx_clear();

858      sim_send_line("led off");

859      sim_run(5u);

860      CHECK(sim_tx_has("OK LED OFF") && sim.led_level == 0u, "lower-case 'led off' accepted, LED off");

861  }

862

863  static void tc04_brightness(void)

864  {

865      tc_begin("TC-04", "BRIGHT command and argument validation");

866      sim_send_line("LED ON");

867      sim_tx_clear();

868      sim_send_line("BRIGHT 40");

869      sim_run(5u);

870      CHECK(sim_tx_has("OK BRIGHT 40") && sim.led_level == 40u, "BRIGHT 40 -> LED level 40 %");

871      sim_tx_clear(); sim_send_line("BRIGHT 0");   sim_run(3u);

872      CHECK(sim_tx_has("OK BRIGHT 0") && sim.led_level == 0u, "BRIGHT 0 accepted (lower bound)");

873      sim_tx_clear(); sim_send_line("BRIGHT 100"); sim_run(3u);

874      CHECK(sim_tx_has("OK BRIGHT 100") && sim.led_level == 100u, "BRIGHT 100 accepted (upper bound)");

875      sim_tx_clear(); sim_send_line("BRIGHT 101");

876      CHECK(sim_tx_has("ERR RANGE 0-100") && brightness == 100u, "BRIGHT 101 rejected, value unchanged");

877      sim_tx_clear(); sim_send_line("BRIGHT -1");

878      CHECK(sim_tx_has("ERR RANGE 0-100"), "BRIGHT -1 rejected");

879      sim_tx_clear(); sim_send_line("BRIGHT abc");

880      CHECK(sim_tx_has("ERR BAD ARG"), "BRIGHT abc -> ERR BAD ARG");

881      sim_tx_clear(); sim_send_line("BRIGHT 5x");

882      CHECK(sim_tx_has("ERR BAD ARG"), "BRIGHT 5x (trailing junk) -> ERR BAD ARG");

883      sim_tx_clear(); sim_send_line("BRIGHT");

884      CHECK(sim_tx_has("ERR MISSING ARG"), "BRIGHT without value -> ERR MISSING ARG");

885  }

886

887  static void tc05_blink(void)

888  {

889      tc_begin("TC-05", "Blink timing (timer interrupt accuracy)");

890      sim_tx_clear();

891      sim_edge_reset();                            /* capture edges from before the command */

892      sim_send_line("LED BLINK 200");

893      CHECK(sim_tx_has("OK LED BLINK 200"), "LED BLINK 200 accepted");

894      sim_run(2200u);

895      CHECK(sim.rise_n >= 10u, "at least 10 blink cycles in 2.2 s");

896      CHECK(all_rise_intervals(200u, 0u), "every rising-edge interval is exactly 200 ms");

897      bool duty_ok = (sim.fall_n >= 10u);

898      for (uint32_t i = 0u; duty_ok && i < 10u; i++) {

899          if (sim.fall_t[i] - sim.rise_t[i] != 100u) { duty_ok = false; }

900      }

901      CHECK(duty_ok, "ON time is exactly 100 ms in each cycle (50 % duty)");

902      sim_tx_clear(); sim_send_line("LED BLINK 49");

903      CHECK(sim_tx_has("ERR RANGE 50-5000"), "period 49 ms rejected (below minimum)");

904      sim_tx_clear(); sim_send_line("LED BLINK 5001");

905      CHECK(sim_tx_has("ERR RANGE 50-5000"), "period 5001 ms rejected (above maximum)");

906      sim_tx_clear(); sim_send_line("LED BLINK");

907      CHECK(sim_tx_has("OK LED BLINK 500"), "LED BLINK without argument uses default 500 ms");

908      sim_send_line("LED BLINK 50");

909      sim_edge_reset();

910      sim_run(500u);

911      CHECK(all_rise_intervals(50u, 0u), "minimum period 50 ms generated correctly");

912  }

913

914  static void tc06_breathe(void)

915  {

916      tc_begin("TC-06", "Breathe (triangle PWM fade)");

917      sim_send_line("LED BREATHE");

918      sim_stats_reset();

919      sim.hist_n = 0u; sim.hist_on = true;

920      sim_run(2000u);

921      sim.hist_on = false;

922      uint32_t i_max = 0u;

923      for (uint32_t i = 0u; i < sim.hist_n; i++) { if (sim.hist[i] > sim.hist[i_max]) { i_max = i; } }

924      CHECK(sim.led_max == 100u, "peak brightness reaches 100 %");

925      CHECK(sim.led_min == 0u, "fade returns to 0 %");

926      CHECK(i_max >= 990u && i_max <= 1010u, "peak occurs half-way through the 2 s cycle");

927      bool up = true, down = true;

928      for (uint32_t i = 1u; i <= i_max; i++) { if (sim.hist[i] < sim.hist[i - 1u]) { up = false; } }

929      for (uint32_t i = i_max + 1u; i < sim.hist_n; i++) { if (sim.hist[i] > sim.hist[i - 1u]) { down = false; } }

930      CHECK(up, "level rises monotonically to the peak");

931      CHECK(down, "level falls monotonically after the peak");

932      sim_send_line("BRIGHT 50");

933      sim_stats_reset();

934      sim_run(2000u);

935      CHECK(sim.led_max == 50u, "BRIGHT 50 scales the breathe peak to 50 %");

936  }

937

938  static void tc07_robust_input(void)

939  {

940      tc_begin("TC-07", "Input robustness: unknown, blank, tabs, backspace, too many args");

941      sim_tx_clear(); sim_send_line("FOO");

942      CHECK(sim_tx_has("ERR UNKNOWN CMD"), "unknown command -> ERR UNKNOWN CMD");

943      sim_tx_clear(); sim_uart_send("\r\n\r\n"); sim_run(5u);

944      CHECK(sim.tx_len == 0u, "blank lines produce no reply");

945      sim_tx_clear(); sim_send_line("LED\tOFF");

946      CHECK(sim_tx_has("OK LED OFF"), "tab used as separator accepted");

947      sim_tx_clear(); sim_send_line("   led    on   ");

948      CHECK(sim_tx_has("OK LED ON"), "extra spaces ignored");

949      sim_tx_clear(); sim_uart_send("LEDD\x08 ON\r\n"); sim_run(3u);

950      CHECK(sim_tx_has("OK LED ON"), "backspace editing works (LEDD<BS> ON -> LED ON)");

951      sim_tx_clear(); sim_send_line("LED BLINK 100 200");

952      CHECK(sim_tx_has("ERR TOO MANY ARGS"), "LED BLINK 100 200 (extra argument) -> ERR TOO MANY ARGS");

953      sim_tx_clear(); sim_send_line("LED A B C D");

954      CHECK(sim_tx_has("ERR TOO MANY ARGS"), "four tokens -> ERR TOO MANY ARGS");

955      sim_tx_clear(); sim_send_line("LED");

956      CHECK(sim_tx_has("ERR MISSING ARG"), "LED without sub-command -> ERR MISSING ARG");

957      sim_tx_clear(); sim_send_line("LED SPIN");

958      CHECK(sim_tx_has("ERR BAD ARG"), "LED SPIN -> ERR BAD ARG");

959  }

960

961  static void tc08_overflow(void)

962  {

963      tc_begin("TC-08", "UART overflow and over-long line handling");

964      char junk[101];

965      memset(junk, 'A', 100u);

966      junk[100] = '\0';

967      sim_tx_clear();

968      sim_uart_burst(junk);                        /* 100 bytes while main loop is busy */

969      CHECK(rx_overflow == 100u - (UART_RX_BUF_SIZE - 1u), "37 bytes dropped and counted (63 fit in the ring)");

970      sim_run(3u);                                 /* main loop drains the buffer */

971      sim_uart_send("\r\n");

972      sim_run(3u);

973      CHECK(sim_tx_has("ERR LINE TOO LONG"), "over-long line rejected with ERR LINE TOO LONG");

974      sim_tx_clear(); sim_send_line("LED ON");

975      sim_run(3u);

976      CHECK(sim_tx_has("OK LED ON") && sim.led_level == 100u, "system fully usable after the overflow");

977      sim_tx_clear(); sim_send_line("STATUS");

978      CHECK(sim_tx_has("rxovf=37"), "STATUS reports rx overflow counter 37");

979      char exact[CMD_MAX_LEN + 2u];                /* exactly CMD_MAX_LEN characters is still legal */

980      snprintf(exact, sizeof exact, "LED ON");

981      memset(exact + 6, ' ', CMD_MAX_LEN - 6u);

982      exact[CMD_MAX_LEN] = '\0';

983      sim_tx_clear(); sim_send_line(exact);

984      CHECK(sim_tx_has("OK LED ON"), "a line of exactly 40 characters is accepted");

985  }

986

987  static void tc09_temperature(void)

988  {

989      tc_begin("TC-09", "Temperature measurement accuracy (ADC + filter)");

990      static const int32_t pts[] = { 0, 250, 375, 500, 990 };

991      for (unsigned k = 0u; k < sizeof pts / sizeof pts[0]; k++) {

992          sim.temp_c10 = pts[k];

993          sim_run(1000u);

994          sim_tx_clear(); sim_send_line("TEMP");

995          int32_t got = -999;

996          bool ok = sim_read_temp_reply(&got);

997          char msg[96];

998          snprintf(msg, sizeof msg, "true %ld.%ld degC -> reported within +-0.2 degC (got %ld.%ld)",

999                   (long)(pts[k] / 10), (long)(pts[k] % 10), (long)(got / 10), (long)(got % 10));

1000          CHECK(ok && got >= pts[k] - 2 && got <= pts[k] + 2, msg);

1001      }

1002  }

1003

1004  static void tc10_spike(void)

1005  {

1006      tc_begin("TC-10", "Moving average suppresses a single-sample glitch");

1007      sim_run(1500u);

1008      sim.spike_raw = 745;                          /* one conversion reads 60.0 degC */

1009      int32_t peak = 0;

1010      for (int i = 0; i < 400; i++) {

1011          sim_step();

1012          if (t_avg_c10 > peak) { peak = t_avg_c10; }

1013      }

1014      CHECK(peak >= 285 && peak <= 305, "glitch is visible but divided by the window (peak about 29.4 degC)");

1015      CHECK(!alert_active && !sim_tx_has("ALERT ON"), "no false alert from a one-sample 60 degC glitch");

1016  }

1017

1018  static void tc11_alert(void)

1019  {

1020      tc_begin("TC-11", "Over-temperature alert, LED override and hysteresis");

1021      sim_run(1200u);

1022      CHECK(!alert_active, "no alert at 25.0 degC");

1023      sim.temp_c10 = 400;                           /* 40.0 degC, threshold 35 */

1024      sim_run(1000u);

1025      CHECK(alert_active && sim_tx_has("ALERT ON temp="), "ALERT ON raised within 1 s of reaching 40 degC");

1026      sim_edge_reset();

1027      sim_run(1000u);

1028      CHECK(all_rise_intervals(ALERT_BLINK_MS, 0u) && sim.rise_n >= 9u,

1029            "LED forced to 100 ms blink during alert (user mode was OFF)");

1030      sim.temp_c10 = 340;                           /* inside hysteresis band (33.0..35.0) */

1031      sim_tx_clear();

1032      sim_run(2000u);

1033      CHECK(alert_active && !sim_tx_has("ALERT OFF"), "34.0 degC is inside the hysteresis band: alert stays on");

1034      sim.temp_c10 = 320;                           /* below 33.0 */

1035      sim_run(1500u);

1036      CHECK(!alert_active && sim_tx_has("ALERT OFF temp="), "ALERT OFF once temperature drops below 33.0 degC");

1037      sim_run(200u);

1038      CHECK(sim.led_level == 0u, "LED returns to the user mode (OFF) after the alert");

1039      sim.temp_c10 = 345;                           /* just under threshold: must not trigger */

1040      sim_tx_clear();

1041      sim_run(2000u);

1042      CHECK(!alert_active, "34.5 degC (just below 35.0) does not trigger");

1043      /* exact boundary behaviour, measured with a noise-free ADC */

1044      sim.noise = false;

1045      sim.temp_c10 = 349;  sim_run(1500u);

1046      CHECK(!alert_active, "34.9 degC (noise-free): no alert");

1047      sim.temp_c10 = 350;  sim_run(1500u);

1048      CHECK(alert_active, "exactly 35.0 degC raises the alert (>= comparison)");

1049      sim.temp_c10 = 331;  sim_run(1500u);

1050      CHECK(alert_active, "33.1 degC is still inside the hysteresis band");

1051      sim.temp_c10 = 330;  sim_run(1500u);

1052      CHECK(!alert_active, "exactly 33.0 degC clears the alert (<= comparison)");

1053  }

1054

1055  static void tc12_threshold(void)

1056  {

1057      tc_begin("TC-12", "THRESH command");

1058      sim.temp_c10 = 300;

1059      sim_run(1500u);

1060      CHECK(!alert_active, "30.0 degC, default threshold 35: no alert");

1061      sim_tx_clear(); sim_send_line("THRESH 28");

1062      CHECK(sim_tx_has("OK THRESH 28"), "THRESH 28 accepted");

1063      sim_run(500u);

1064      CHECK(alert_active, "alert raised after threshold lowered to 28 degC");

1065      sim_tx_clear(); sim_send_line("THRESH 9");

1066      CHECK(sim_tx_has("ERR RANGE 10-100") && thresh_c == 28, "THRESH 9 rejected, old value kept");

1067      sim_tx_clear(); sim_send_line("THRESH 101");

1068      CHECK(sim_tx_has("ERR RANGE 10-100"), "THRESH 101 rejected");

1069      sim_tx_clear(); sim_send_line("THRESH hot");

1070      CHECK(sim_tx_has("ERR RANGE 10-100"), "THRESH hot rejected");

1071      sim_tx_clear(); sim_send_line("THRESH");

1072      CHECK(sim_tx_has("ERR MISSING ARG"), "THRESH without value -> ERR MISSING ARG");

1073  }

1074

1075  static void tc13_status(void)

1076  {

1077      tc_begin("TC-13", "STATUS report and TEMP before first sample");

1078      sim.noise = false;                            /* noise-free ADC gives an exact reading */

1079      sim_tx_clear(); sim_send_line("TEMP");

1080      CHECK(sim_tx_has("ERR NO DATA"), "TEMP before the first sample -> ERR NO DATA");

1081      sim_run(1000u);

1082      sim_send_line("LED BLINK 300");

1083      sim_send_line("BRIGHT 70");

1084      sim_send_line("THRESH 30");

1085      sim_tx_clear(); sim_send_line("STATUS");

1086      CHECK(sim_tx_has("mode=BLINK") && sim_tx_has("period=300"), "STATUS shows mode and period");

1087      CHECK(sim_tx_has("bright=70") && sim_tx_has("thresh=30C"), "STATUS shows brightness and threshold");

1088      CHECK(sim_tx_has("temp=25.0C") && sim_tx_has("alert=0"), "STATUS shows temperature and alert flag");

1089      CHECK(sim_tx_has("rxovf=0") && sim_tx_has("tickovr=0") && sim_tx_has("wdg_resets=0"), "STATUS shows health counters");

1090  }

1091

1092  static void tc14_watchdog(void)

1093  {

1094      tc_begin("TC-14", "Watchdog recovers a hung main loop");

1095      sim_send_line("LED ON");

1096      sim_run(100u);

1097      sim_tx_clear();

1098      sim_send_line("HANG");

1099      uint32_t t_hang = sim.t_ms;

1100      CHECK(sim_tx_has("OK HANG") && g_hung, "HANG command stops the main loop");

1101      sim_tx_clear();

1102      sim_run(1900u);

1103      CHECK(!sim_tx_has("BOOT LED-CTRL"), "no reset before the 2 s timeout");

1104      CHECK(sim.persist == 0u, "reset counter still 0");

1105      sim_send_line("STATUS");                      /* ignored: loop is hung */

1106      CHECK(!sim_tx_has("STATUS mode"), "hung firmware ignores UART commands");

1107      sim_run(300u);

1108      CHECK(sim_tx_has("BOOT LED-CTRL v" FW_VERSION " reset=WATCHDOG resets=1"), "watchdog reset happened and was reported");

1109      uint32_t dt = sim.last_reset_t - t_hang;

1110      CHECK(dt >= 1990u && dt <= 2010u, "reset occurred about 2000 ms after the hang");

1111      CHECK(!g_hung && user_mode == LED_OFF, "firmware restarted in its default state");

1112      sim_tx_clear();

1113      sim_send_line("STATUS");

1114      CHECK(sim_tx_has("wdg_resets=1"), "reset counter persisted in the backup register and is reported");

1115      sim_tx_clear(); sim_send_line("LED ON");

1116      CHECK(sim_tx_has("OK LED ON"), "console works again after recovery");

1117  }

1118

1119  static void tc15_ringbuffer(void)

1120  {

1121      tc_begin("TC-15", "Ring buffer unit test (wrap-around, full, empty)");

1122      uint8_t out = 0u;

1123      CHECK(!rx_pop(&out), "empty buffer: pop fails");

1124      bool order = true;

1125      for (unsigned i = 0u; i < 500u; i++) {        /* wrap the 64-byte buffer many times */

1126          sim.rdr = (uint8_t)i;

1127          USART2_IRQHandler();

1128          if (!rx_pop(&out) || out != (uint8_t)i) { order = false; }

1129      }

1130      CHECK(order && rx_overflow == 0u, "500 bytes pushed/popped in order across many wrap-arounds");

1131      for (unsigned i = 0u; i < UART_RX_BUF_SIZE + 10u; i++) { sim.rdr = (uint8_t)i; USART2_IRQHandler(); }

1132      CHECK(rx_overflow == 11u, "pushing 74 bytes into a 64-slot ring drops exactly 11 (one slot is the full marker)");

1133      unsigned cnt = 0u;

1134      bool first_ok = true;

1135      while (rx_pop(&out)) { if (cnt < 63u && out != (uint8_t)cnt) { first_ok = false; } cnt++; }

1136      CHECK(cnt == 63u && first_ok, "63 oldest bytes are preserved intact");

1137  }

1138

1139  static void tc16_adc(void)

1140  {

1141      tc_begin("TC-16", "ADC conversion unit test");

1142      CHECK(adc_to_c10(0u) == 0, "code 0 -> 0.0 degC");

1143      CHECK(adc_to_c10(310u) == 250, "code 310 -> 25.0 degC");

1144      CHECK(adc_to_c10(4095u) == 3300, "full scale 4095 -> 330.0 degC (3.3 V at 10 mV/degC)");

1145      CHECK(adc_to_c10(745u) == 600, "code 745 -> 60.0 degC");

1146      char tb[16];

1147      CHECK(strcmp(fmt_c10(-35, tb, sizeof tb), "-3.5") == 0, "formatter prints negative values (-3.5)");

1148      CHECK(strcmp(fmt_c10(5, tb, sizeof tb), "0.5") == 0, "formatter prints 0.5");

1149  }

1150

1151  static int run_selftest(void)

1152  {

1153      printf("LED-CTRL firmware self-test (simulated target)\n");

1154      printf("==============================================\n");

1155      tc01_boot();        tc02_timebase();   tc03_basic_commands(); tc04_brightness();

1156      tc05_blink();       tc06_breathe();    tc07_robust_input();   tc08_overflow();

1157      tc09_temperature(); tc10_spike();      tc11_alert();          tc12_threshold();

1158      tc13_status();      tc14_watchdog();   tc15_ringbuffer();     tc16_adc();

1159      printf("\n==============================================\n");

1160      printf("RESULT: %d checks passed, %d failed -> %s\n", g_pass, g_fail, g_fail == 0 ? "ALL PASS" : "FAILURES");

1161      return g_fail == 0 ? 0 : 1;

1162  }

1163

1164  /* =============================================================================

1165   *  SECTION G : DEMO SCENARIO AND INTERACTIVE MODE

1166   * ============================================================================= */

1167  static void ramp_temp(int32_t from_c10, int32_t to_c10, int32_t step_c10, uint32_t ms_per_step)

1168  {

1169      int32_t t = from_c10;

1170      while ((step_c10 > 0) ? (t < to_c10) : (t > to_c10)) {

1171          t += step_c10;

1172          sim.temp_c10 = t;

1173          sim_run(ms_per_step);

1174      }

1175  }

1176

1177  static void run_demo(const char *csv_path)

1178  {

1179      sim_init();

1180      sim.echo = true;

1181      if (csv_path != NULL) {

1182          sim.csv = fopen(csv_path, "w");

1183          if (sim.csv != NULL) { fprintf(sim.csv, "t_ms,led_pct,temp_true_c10,temp_avg_c10,alert\n"); }

1184      }

1185      printf("--- DEMO: scripted scenario on the simulated board ---\n");

1186      fw_boot(hal_reset_cause());

1187      sim_run(500u);

1188      sim_send_line("HELP");

1189      sim_send_line("LED BLINK 400");   sim_run(2000u);

1190      sim_send_line("LED BREATHE");     sim_run(3000u);

1191      sim_send_line("BRIGHT 60");       sim_run(1000u);

1192      sim_send_line("LED ON");          sim_run(500u);

1193      sim_send_line("LED OFF");         sim_run(500u);

1194      sim_send_line("TEMP");

1195      sim_send_line("LED BLINK 1000");

1196      sim_send_line("BRIGHT 200");      /* deliberate error */

1197      sim_send_line("FLY");             /* deliberate error */

1198      sim_send_line("STATUS");

1199      printf("--- temperature rises 25 -> 40 degC (threshold 35) ---\n");

1200      ramp_temp(250, 400, 5, 200u);

1201      sim_run(1000u);

1202      sim_send_line("STATUS");

1203      printf("--- temperature falls 40 -> 28 degC ---\n");

1204      ramp_temp(400, 280, -5, 200u);

1205      sim_run(1000u);

1206      sim_send_line("STATUS");

1207      printf("--- watchdog demonstration ---\n");

1208      sim_send_line("HANG");

1209      sim_run(2600u);

1210      sim_send_line("STATUS");

1211      printf("--- end of demo (simulated time %lu ms) ---\n", (unsigned long)sim.t_ms);

1212      if (sim.csv != NULL) { fclose(sim.csv); sim.csv = NULL; }

1213  }

1214

1215  static void run_interactive(void)

1216  {

1217      char buf[256];

1218      sim_init();

1219      sim.echo = true;

1220      fw_boot(hal_reset_cause());

1221      printf("Interactive mode. Type UART commands. Directives: @wait <ms>, @temp <degC>, @quit\n");

1222      while (fgets(buf, sizeof buf, stdin) != NULL) {

1223          size_t n = strlen(buf);

1224          while (n > 0u && (buf[n - 1u] == '\n' || buf[n - 1u] == '\r')) { buf[--n] = '\0'; }

1225          if (strcmp(buf, "@quit") == 0) { break; }

1226          unsigned ms = 0u;

1227          double   deg = 0.0;

1228          if (sscanf(buf, "@wait %u", &ms) == 1) { sim_run(ms); continue; }

1229          if (sscanf(buf, "@temp %lf", &deg) == 1) {

1230              sim.temp_c10 = (int32_t)(deg * 10.0 + (deg >= 0 ? 0.5 : -0.5));

1231              printf("[%7lu ms] SIM> temperature set to %.1f degC\n", (unsigned long)sim.t_ms, deg);

1232              continue;

1233          }

1234          sim_send_line(buf);

1235          sim_run(50u);

1236      }

1237  }

1238

1239  int main(int argc, char **argv)

1240  {

1241      bool selftest = false, demo = false, inter = false, verbose = false;

1242      const char *csv = NULL;

1243      for (int i = 1; i < argc; i++) {

1244          if (strcmp(argv[i], "--selftest") == 0)         { selftest = true; }

1245          else if (strcmp(argv[i], "--demo") == 0)        { demo = true; }

1246          else if (strcmp(argv[i], "--interactive") == 0) { inter = true; }

1247          else if (strcmp(argv[i], "--verbose") == 0)     { verbose = true; }

1248          else if (strcmp(argv[i], "--csv") == 0 && i + 1 < argc) { csv = argv[++i]; }

1249          else {

1250              fprintf(stderr, "unknown option '%s'\n", argv[i]);

1251              return 2;

1252          }

1253      }

1254      if (!selftest && !demo && !inter) {

1255          printf("usage: %s [--selftest [--verbose]] [--demo [--csv file]] [--interactive]\n", argv[0]);

1256          return 2;

1257      }

1258      if (selftest) {

1259          sim.echo = verbose;

1260          return run_selftest();

1261      }

1262      if (demo)  { run_demo(csv); return 0; }

1263      run_interactive();

1264      return 0;

1265  }